2.2.7.  Lock

The MPLS-TP OAM toolset MAY provide a function enabling to

administratively shut down a PW, LSP or Section; that is, to stop

user traffic being sent over that PW, LSP or Section.

The lock function MAY be applied to a unidirectional or bidirectional PW, LSP or Section.  If the lock function is applied to bidirectional a PW, LSP or Section then it SHOULD be possible to cause it to be applied to both directions.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

2.2.8.  Alarm Notification

MB> We should break this into several independent requirements: notification of Lock: Local/remote fault causing loss of CC: Wrong CV received <not sure if we need this, its implicit in 2.2.3>.

2.2.8a Notification of Lock

The MPLS-TP OAM toolset MUST provide a function to enable the

End Point at the ingress to notify

the End Points at the egress that the lock function is active.  The egress end points MUST notify the local client layer that the server layer PW, LSP or Section has been locked.

MB> How this is used is out of scope for this draft.

The MPLS-TP OAM toolset MUST allow for the distinction between a

fault condition and an administrative locking action.

MB> Not sure we need this if we separate the requirements.

The server layer End Points generating the notification and the

client layer End Points receiving the notification may or may not be

the same nodes.  A mechanism MUST be provided to support both cases.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between the End Points of PWs, LSPs

and Sections and the End Points of the PWs and/or LSPs affected by

the administrative locking.

2.2.8b Differentiation between local and remote faults.

When an MPLS-TP or PW end point detects a continuity check defect it MUST provide a local indication to any clients and the management function. It MUST also indicate if the fault causing this is local (i.e. within the node containing the end point) or remote.

2.2.8c Connectivity verification failure

When an MPLS-TP or PW end point detects a continuity verification defect it MUST provide a local indication to any clients and the management function. It MUST also provide the capability to shut down the PW, LSP or Section; that is, stop user traffic being sent from that PW, LSP or Section.

2.2.9.  Client Failure Indication

The MPLS-TP OAM toolset MUST provide a function to enable the

propagation of client fault condition detected at the ingress to a PW, LSP or Section, across the MPLS-TP

network

.

MB> The requirement to support the tool should be client independent, we can, on a case by case basis for each client decide if we will use this tool.  My vote would be to always use this tool since it provides consistent client independent operation.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.