MPLS Working Group                                     M. Vigoureux, Ed.

Internet-Draft                                            Alcatel-Lucent

Intended status: Informational                              D. Ward, Ed.

Expires: September 10, 2009                          Cisco Systems, Inc.

M. Betts, Ed.

Nortel Networks

March 9, 2009

Requirements for OAM in MPLS Transport Networks

draft-ietf-mpls-tp-oam-requirements-01

Status of this Memo

This Internet-Draft is submitted to IETF in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups.  Note that

other groups may also distribute working documents as Internet-

Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress".

The list of current Internet-Drafts can be accessed at

http://www.ietf.org/ietf/1id-abstracts.txt.

The list of Internet-Draft Shadow Directories can be accessed at

http://www.ietf.org/shadow.html.

This Internet-Draft will expire on September 10, 2009.

Copyright Notice

Copyright (c) 2009 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents in effect on the date of

publication of this document (http://trustee.ietf.org/license-info).

Please review these documents carefully, as they describe your rights

and restrictions with respect to this document.

Vigoureux, et al.      Expires September 10, 2009               [Page 1]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

Abstract

This document lists the requirements for the Operations,

Administration and Maintenance functionality of MPLS Transport

Profile.  These requirements apply to pseudowires, Label Switched

Paths, and Sections.  Architectural, functional and operational

requirements are covered in this document.

Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in RFC 2119 [1].

Vigoureux, et al.      Expires September 10, 2009               [Page 2]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

Table of Contents

1.  Introduction . . . . . . . . . . . . . . . . . . . . . . . . .  4

1.1.  Definitions  . . . . . . . . . . . . . . . . . . . . . . .  5

1.2.  Contributing Authors . . . . . . . . . . . . . . . . . . .  5

2.  OAM Requirements . . . . . . . . . . . . . . . . . . . . . . .  5

2.1.  Architectural Requirements . . . . . . . . . . . . . . . .  6

2.1.1.  Independence . . . . . . . . . . . . . . . . . . . . .  6

2.1.2.  Addressing, Routing and Forwarding . . . . . . . . . .  6

2.1.3.  Interoperability and Interworking  . . . . . . . . . .  6

2.1.4.  Data Plane . . . . . . . . . . . . . . . . . . . . . .  7

2.1.5.  Scope  . . . . . . . . . . . . . . . . . . . . . . . .  7

2.2.  Functional Requirements  . . . . . . . . . . . . . . . . .  7

2.2.1.  General Requirements . . . . . . . . . . . . . . . . .  8

2.2.2.  Continuity Checks  . . . . . . . . . . . . . . . . . .  8

2.2.3.  Connectivity Verifications . . . . . . . . . . . . . .  9

2.2.4.  Diagnostic . . . . . . . . . . . . . . . . . . . . . .  9

2.2.5.  Adjacency  . . . . . . . . . . . . . . . . . . . . . .  9

2.2.6.  Route Tracing  . . . . . . . . . . . . . . . . . . . . 10

2.2.7.  Lock . . . . . . . . . . . . . . . . . . . . . . . . . 10

2.2.8.  Alarm Notification . . . . . . . . . . . . . . . . . . 10

2.2.9.  Client Failure Indication  . . . . . . . . . . . . . . 11

2.2.10. Remote Defect Indication . . . . . . . . . . . . . . . 11

2.2.11. Packet Loss  . . . . . . . . . . . . . . . . . . . . . 11

2.2.12. Delay Measurement  . . . . . . . . . . . . . . . . . . 12

2.3.  Operational Requirements . . . . . . . . . . . . . . . . . 12

3.  Congestion Considerations  . . . . . . . . . . . . . . . . . . 14

4.  Security Considerations  . . . . . . . . . . . . . . . . . . . 14

5.  IANA Considerations  . . . . . . . . . . . . . . . . . . . . . 14

6.  Acknowledgements . . . . . . . . . . . . . . . . . . . . . . . 14

7.  References . . . . . . . . . . . . . . . . . . . . . . . . . . 15

7.1.  Normative References . . . . . . . . . . . . . . . . . . . 15

7.2.  Informative References . . . . . . . . . . . . . . . . . . 15

Authors' Addresses . . . . . . . . . . . . . . . . . . . . . . . . 16

Vigoureux, et al.      Expires September 10, 2009               [Page 3]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

1.  Introduction

In the context of MPLS Transport Profile (MPLS-TP, see [5] and [6]),

the rationale for Operations, Administration and Maintenance (OAM)

mechanisms are twofold as they can serve:

o  as a network-oriented mechanism (used by a transport network

operator) to monitor his network infrastructure and to implement

internal mechanisms in order to enhance the general behaviour and

the level of performance of his network (e.g., protection

mechanism in case of node or link failure).  For example, fault

localization is typically associated with this use case.

o  as a service-oriented mechanism (used by a transport service

provider) to monitor services offered to end customers in order to

be able to react rapidly in case of a problem and to be able to

verify some of the Service Level Agreement (SLA) parameters

(e.g., using performance monitoring) negotiated with the end

customer.  Note that a transport service could be provided over

several networks or administrative domains that may not all be

owned and managed by the same transport service provider.

More generally, OAM is an important and fundamental functionality in

transport networks as it contributes to:

o  the reduction of operational complexity and costs, by allowing

efficient and automatic detection, localisation, handling, and

diagnosis of defects, and by minimizing service interruptions and

operational repair times.

o  the enhancement of network availability, by ensuring that defects,

for example resulting in misdirected customer traffic, and faults,

are detected, diagnosed and dealt with before a customer reports

the problem.

o  meet service and performance objectives, by running OAM

functionality which allows SLA verification in a multi-maintenance

domain environment and allow the determination of service

degradation due, for example, to packet delay or packet loss.

This document lists the requirements for the OAM functionality of

MPLS-TP.  These requirements apply to pseudowires (PWs), Label

Switched Paths (LSPs), and Sections.

These requirements are derived from a set of requirements specified

by ITU-T and first published in the ITU-T Supplement Y.Sup4 [7].

By covering transport specificities, these requirements

Vigoureux, et al.      Expires September 10, 2009               [Page 4]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

complement those identified in RFC 4377 [8].

1.1.  Definitions

In this document we refer to a fault as the inability of a function

to perform a required action.  This does not include an inability due

to preventive maintenance, lack of external resources, or planned

actions.  See also ITU-T G.806 [2].

In this document we refer to a defect as the situation in which the

density of anomalies has reached a level where the ability to perform

a required function has been interrupted.  See also ITU-T G.806 [2].

In this document we refer to a Label Edge Router (LER), for a given

LSP or Section, and to a PW Terminating Provider Edge (T-PE), for a

given PW, as an End Point.  Further, we refer to a Label Switching

Router (LSR), for a given LSP, and to a PW Switching Provider Edge

(S-PE), for a given PW, as an Intermediate Point.  This document does

not make any distinction between End Points (e.g., source and

destination) as it can be inferred from the context of the sentences.

In this document we use the term "node" as a general reference to End

Points and Intermediate Points.

Other definitions, relating to MPLS-TP, can be found in [6].

1.2.  Contributing Authors

The editors gratefully acknowledge the contributions of Matthew

Bocci, Italo Busi, Thomas Dietz, Huub van Helvoort, Wataru Imajuku,

Marc Lasserre, Lieven Levrau, Han Li, Julien Meuric, Philippe Niger,

Benjamin Niven-Jenkins, Jing Ruiquan, Nurit Sprecher, Yuji Tochio,

Satoshi Ueno and Yaacov Weingarten.

2.  OAM Requirements

This section lists the requirements by which the OAM functionality of

MPLS-TP should abide.  Note that some requirements for this

application of MPLS are similar to some of those listed in RFC 4377

[8].

The requirements listed below may be met by one or more OAM

protocols; the definition or selection of these protocols is outside

the scope of this document.

Vigoureux, et al.      Expires September 10, 2009               [Page 5]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

2.1.  Architectural Requirements

2.1.1.  Independence

OAM functions SHOULD be independent of the underlying tunnelling or

point-to-point technology or transmission media.

OAM functions SHOULD be independent of the service a PW may emulate.

The set of OAM functions operated on a PW, LSP or Section SHOULD be

independent of the set of OAM functions operated on a different PW,

LSP or Section.  In other words, only the OAM functions available for

e.g., a LSP, should be used to achieve the OAM objectives for that

LSP.  Note that independence should not be understood here in terms

of isolation as there can be interactions between OAM functions

operated on e.g., a LSP and on another LSP or on a PW.

OAM functions MUST operate and be configurable even in the absence of

a control plane.  Conversely, OAM functions SHOULD be configurable as

part of connectivity (e.g., LSP or PW) management.  Means for

configuring OAM functions and for connectivity management are outside

the scope of this document.

2.1.2.  Addressing, Routing and Forwarding

The OAM functionality may be deployed in a variety of environments.

o  In some environments (e.g., IP/MPLS environments), IP routing and

forwarding capabilities are inherently present.  In this case, the

OAM functionality MUST support the use of IP routing and

forwarding capabilities.

o  In some environments (e.g., MPLS-TP environments), IP routing and

forwarding capabilities may not necessarily be present.  In this

case, the OAM functions and their operation MUST NOT require

relying on IP routing and forwarding capabilities.

In case OAM messages need to incorporate identification information

(e.g., of source and/or destination nodes), the protocol solution

MUST, at least, support an IP addressing structure and MUST also be

extensible to support additional addressing schemes.

2.1.3.  Interoperability and Interworking

It is REQUIRED by this document that OAM interoperability is achieved

across the environments described in Section 2.1.2.  It is also

REQUIRED by this document that the two first requirements of Section

2.1.2 still hold and MUST still be met when interoperability is

Vigoureux, et al.      Expires September 10, 2009               [Page 6]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

achieved.

When MPLS-TP is run with IP routing and forwarding capabilities, it

MUST be possible to operate any of the existing IP/MPLS and PW OAM

functionalities (e.g., LSP-Ping [3], MPLS-BFD [9], VCCV [4] and VCCV-

BFD [10]).

The protocol solution(s) developed to meet the requirements listed in

this document MUST interwork with the existing IP/MPLS and PW OAM

protocols.

2.1.4.  Data Plane

OAM functions operate in the data plane.  OAM packets MUST run in-

band; that is, OAM packets for a specific PW, LSP or Section MUST

follow the exact same data path as user traffic of that PW, LSP or

Section.

It MUST be possible to discriminate user traffic from OAM packets.

This includes a means to differentiate OAM packets from user traffic

as well as the capability to apply specific treatment, to OAM

packets, at the nodes targeted by these OAM packets.

As part of the design of OAM protocol solutions for MPLS-TP, a

mechanism enabling to encapsulate and differentiate OAM messages, on

a PW, LSP or Section, MUST be provided.  Such mechanism MUST also

support the encapsulation and differentiation of existing IP/MPLS and

PW OAM messages.

2.1.5.  Scope

The service emulated by a single segment or a multi-segment PW may

span multiple domains.  A LSP may also span multiple domains.  It

MUST be possible to perform OAM functions on a per domain basis and

across multiple domains.  More generally, it MUST be possible to

perform OAM functions between any two switching elements (e.g., LSR

or S-PE) of a LSP or PW.  This is referred to as (concatenated)

segment monitoring.

2.2.  Functional Requirements

Hereafter are listed the required functions composing the MPLS-TP OAM

toolset.  The list may not be exhaustive and as such the OAM

mechanisms developed in support of the identified requirements SHALL

be extensible and thus SHALL NOT preclude the definition of

additional OAM functions, in the future.

The design of OAM mechanisms, for MPLS-TP, MUST support the ability to

Vigoureux, et al.      Expires September 10, 2009               [Page 7]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

use vendor specific and experimental OAM functions.  These

functions MUST be disabled by default.

The use of any OAM function MUST be optional for the service provider

or network operator and a network operator or service provider MUST

be able to choose which OAM function(s) to use and on which PW, LSP

or Section to apply it(them) to.

It is RECOMMENDED by this document that a protocol solution,

realizing a given function, effectively provides a fully featured

function, i.e., a function which is applicable to all the cases

identified in the table in Section 2.3, for that function.

The OAM functions MUST be able to be operated on PWs, LSPs and

Sections.

Note that the functions listed below can be used for fault

management, performance monitoring and/or protection switching

applications.  For example, connectivity verification can be used for

fault management application by detecting failure conditions, but may

also be used for performance monitoring application through its

contribution to the evaluation of performance metrics (e.g.,

unavailability time).  Nevertheless, it is outside the scope of this

document to specify which function should be used for which

application.

2.2.1.  General Requirements

If a defect or fault occurs on a PW, LSP or Section, mechanisms MUST

be provided to detect it, diagnose it, localize it, and notify the

appropriate entities.  Corrective actions SHOULD be taken according

to the type of defect or fault.

Furthermore, in case of a fault or defect, affecting a service

provided by a service provider, mechanisms MUST be available for the

service provider to be informed of the fault or defect even if the

fault or defect is located outside of his domain.

2.2.2.  Continuity Checks

The MPLS-TP OAM toolset MUST provide a function to enable service

providers and network operators to detect loss of continuity, but

also unintended connectivity, on a PW, LSP or Section.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

Vigoureux, et al.      Expires September 10, 2009               [Page 8]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

Means MUST be available to set the frequency at which

this function is

performed as well as to parameterize the criteria, if

any (e.g., number of consecutive OAM messages not received), based on

which loss of continuity or unintended connectivity is detected.  A

default value MAY be defined.

2.2.3.  Connectivity Verifications

The MPLS-TP OAM toolset MUST provide a function to enable service

providers and network operators to verify the connectivity of a PW,

LSP or Section.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points and Intermediate

Points of PWs and LSPs, and between End Points of PWs, LSPs and

Sections.

Note that, this function is sometime referred to as loopback as End

Points expect to receive some level of information as a result of

their action.

2.2.4.  Diagnostic

The MPLS-TP OAM toolset MAY provide a function to enable service

providers and network operators to perform diagnostic tests (e.g.,

verify bandwidth throughput) on a PW, LSP or Section.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points and Intermediate

Points of PWs and LSPs, and between End Points of PWs, LSPs and

Sections.

This function MAY be provided as part of the Connectivity

Verifications function (see Section 2.2.3).

2.2.5.  Adjacency

The MPLS-TP OAM toolset MUST provide a function to enable an End

Point to request, to, and receive from, any node along a PW, LSP or

Section, a certain level of information (e.g., identification,

distance in hops).

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points and any node of

a PW, LSP and Section.

Vigoureux, et al.      Expires September 10, 2009               [Page 9]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

This function MAY be provided jointly with the Route Tracing function

(see Section 2.2.6).

2.2.6.  Route Tracing

The MPLS-TP OAM toolset MUST provide a function to enable service

providers and network operators to trace the route a PW, LSP or

Section.  The information collected SHOULD include identifiers

related to the nodes composing that route and MAY include interface

identifiers.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points and Intermediate

Points of PWs and LSPs, and between End Points of PWs, LSPs and

Sections.

This function MAY be provided jointly with the Adjacency function

(see Section 2.2.5).

2.2.7.  Lock

The MPLS-TP OAM toolset MAY provide a function enabling to

administratively shut down a PW, LSP or Section; that is, to stop

user traffic being sent over that PW, LSP or Section.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

2.2.8.  Alarm Notification

The MPLS-TP OAM toolset MUST provide a function to enable server

layer End Points to notify a fault condition of an administrative

locking to the client layer End Points affected by this status.  This

would enable the client layer to suppress alarms that may be generated

as a result of the fault condition or of the administrative

locking in the server layer.

The MPLS-TP OAM toolset MUST allow for the distinction between a

fault condition and an administrative locking action.

The server layer End Points generating the notification and the

client layer End Points receiving the notification may or may not be

the same nodes.  A mechanism MUST be provided to support both cases.

This function SHOULD be performed pro-actively.

Vigoureux, et al.      Expires September 10, 2009              [Page 10]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

This function SHOULD be performed between the End Points of PWs, LSPs

and Sections and the End Points of the PWs and/or LSPs affected by

the fault condition or administrative locking.

2.2.9.  Client Failure Indication

The MPLS-TP OAM toolset MUST provide a function to enable the

propagation of client fault condition information, across the MPLS-TP

network, if the client layer OAM mechanisms do not provide an alarm

notification/propagation mechanism.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

2.2.10.  Remote Defect Indication

The MPLS-TP OAM toolset MUST provide a function to enable an End

Point to notify its associated End Point of the detection of a fault

or defect that it detects on a PW, LSP or Section between them.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

This function MAY be provided in conjunction with the Continuity Check function (???)

2.2.11.  Packet Loss

Packet loss ratio is the ratio of the user packets not delivered to

the total number of user packets transmitted during a defined time

interval.  The number of user packets not delivered is the difference

between the number of user packets transmitted by an End Point and

the number of user packets received at an End Point.

The MPLS-TP OAM toolset MUST provide a function to enable service

providers and network operators to derive packet loss ratio over a

PW, LSP or Section.

This OAM function MUST support the configurability of the interval of

time during which the measure is performed.

This function SHOULD be performed pro-actively.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

Vigoureux, et al.      Expires September 10, 2009              [Page 11]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

2.2.12.  Delay Measurement

The MPLS-TP OAM toolset MUST provide a function to enable service

providers and network operators to measure the one-way, and if

appropriate, the two-way, delay of a PW, LSP or Section.

o  One-way delay is the time elapsed from the start of transmission

of the first bit of an OAM packet by an End Point until the

reception of the last bit of that OAM packet by the other End

Point.

o  Two-way delay is the time elapsed from the start of transmission

of the first bit of an OAM packet by a End Point until the

reception of the last bit of that OAM packet by the same End

Point, when the loopback is performed at the other End Point.

This function SHOULD be performed on-demand.

This function SHOULD be performed between End Points of PWs, LSPs and

Sections.

2.3.  Operational Requirements

The OAM functions MUST NOT rely on user traffic to achieve their

objectives; that is, dedicated OAM messages MUST be used.

Some OAM functions require certain parameters for their operation.

These parameters MUST be configurable.  A default value MAY be

defined.

The specification of certain parameters' values SHOULD be such that

it accounts, at the design phase, for various possible network

conditions (e.g., the continuity check function should continue to

meet its objective (i.e. detect failures) even in the context of high

traffic load (e.g., congestion)).

This document does not mandate the use of a particular OAM function.

However, it is RECOMMENDED that MPLS-TP enables continuity checks to

be performed on every PW, LSP and Section in order to reliably detect

connectivity defects and faults.

OAM functions MUST be applicable to bidirectional point-to-point PWs,

LSPs and Sections, and a subset of these OAM functions MUST be

applicable to unidirectional point-to-point and point-to-multipoint

PWs, LSPs and Sections.  This subset is based on the nature of both

the OAM functions and the connections to which they can apply.

Vigoureux, et al.      Expires September 10, 2009              [Page 12]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

The following table describes how, between which points of PWs, LSPs

and Sections SHOULD the required OAM functions be applied.  In these

tables U stands for unidirectional; B stands for bidirectional; EP

stands for an OAM function being performed between End Points; IP

stands for an OAM function being performed between End Points and

Intermediate Points.  Crosses (x) indicate the way the considered

function should be applied; numbers indicate the way the considered

function should be applied while pointing to a footnote providing

additional details.

+-------------------------------------------+

|      on-demand      |      pro-active     |

|---------------------+----------+----------|

|    MEP   |   MIP    |    MEP   |   MIP    |

|----------+----------+----------+----------|

| P2P |P2MP| P2P |P2MP| P2P |P2MP| P2P |P2MP|

|-----+----+----------+----------+-----+----|

|U |B | U  |U |B | U  |U |B | U  |U |B | U  |

+----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| c. checks            |  |  |    |  |  |    |x |x | x  |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| c. verifications     |1 |x | 1  |1 |x | 1  |  |  |    |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| diagnostic           |x |x | x  |2 |2 | 2  |  |  |    |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| adjacency            |1 |x | 1  |1 |x | 1  |  |  |    |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| route tracing        |1 |x | 1  |1 |x | 1  |  |  |    |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| lock                 |x |x | x  |  |  |    |  |  |    |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| alarm notification   |  |  |    |  |  |    |x |x | x  |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| client fail. indic.  |  |  |    |  |  |    |2 |x | 2  |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| remote defect indic. |  |  |    |  |  |    |1 |x | 1  |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| packet loss          |2 |3 | 2  |  |  |    |x |4 | x  |  |  |    |

|----------------------+--+--+----+--+--+----+--+--+----+--+--+----|

| delay measurement    |x |x | x  |  |  |    |2 |2 | 2  |  |  |    |

+----------------------+--+--+----+--+--+----+--+--+----+--+--+----+

1: the function MAY be provided if a return path exists

2: the function MAY be performed

3: the function SHOULD be performed in one direction

4: the function SHOULD be performed in both directions

OAM functions and their applicability scope

Vigoureux, et al.      Expires September 10, 2009              [Page 13]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

3.  Congestion Considerations

A mechanism (e.g., rate limiting) MUST be provided to prevent OAM

packets from causing congestion in the PSN.

4.  Security Considerations

This document, as itself, does not imply any security consideration

but OAM, as such, is subject to several security considerations.  OAM

messages can reveal sensitive information such as passwords,

performance data and details about e.g., the network topology.

The nature of OAM therefore suggests having some form of

authentication, authorization and encryption in place.  This will

prevent unauthorized access to MPLS-TP equipment and it will prevent

third parties from learning about sensitive information about the

transport network.

In general, mechanisms SHOULD be provided to ensure that OAM

functions cannot be accessed unauthorized.

Further, OAM messages MAY be authenticated to prove their origin and

to make sure that they are destined for the receiving node.

An OAM packet received over a PW, LSP or Section MUST NOT be

forwarded beyond the End Point of that PW, LSP or Section, so as to

avoid that the OAM packet leaves the current administrative domain.

5.  IANA Considerations

There are no IANA actions required by this draft.

6.  Acknowledgements

The authors would like to thank all members of the teams (the Joint

Working Team, the MPLS Interoperability Design Team in IETF and the

MPLS-TP Ad Hoc Group in ITU-T) involved in the definition and

specification of MPLS-TP.

Vigoureux, et al.      Expires September 10, 2009              [Page 14]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

7.  References

7.1.  Normative References

[1]   Bradner, S., "Key words for use in RFCs to Indicate Requirement

Levels", BCP 14, RFC 2119, March 1997.

[2]   ITU-T Recommendation G.806, "Characteristics of transport

equipment - Description methodology and generic functionality",

2009.

[3]   Kompella, K. and G. Swallow, "Detecting Multi-Protocol Label

Switched (MPLS) Data Plane Failures", RFC 4379, February 2006.

[4]   Nadeau, T. and C. Pignataro, "Pseudowire Virtual Circuit

Connectivity Verification (VCCV): A Control Channel for

Pseudowires", RFC 5085, December 2007.

7.2.  Informative References

[5]   Bocci, M., Bryant, S., and L. Levrau, "A Framework for MPLS in

Transport Networks", draft-ietf-mpls-tp-framework-00 (work in

progress), November 2008.

[6]   Niven-Jenkins, B., Brungard, D., Betts, M., Sprecher, N., and

S. Ueno, "MPLS-TP Requirements",

draft-ietf-mpls-tp-requirements-04 (work in progress),

February 2009.

[7]   ITU-T Supplement Y.Sup4, "ITU-T Y.1300-series: Supplement on

transport requirements for T-MPLS OAM and considerations for

the application of IETF MPLS technology", 2008.

[8]   Nadeau, T., Morrow, M., Swallow, G., Allan, D., and S.

Matsushima, "Operations and Management (OAM) Requirements for

Multi-Protocol Label Switched (MPLS) Networks", RFC 4377,

February 2006.

[9]   Aggarwal, R., Kompella, K., Nadeau, T., and G. Swallow, "BFD

For MPLS LSPs", draft-ietf-bfd-mpls-07 (work in progress),

June 2008.

[10]  Nadeau, T. and C. Pignataro, "Bidirectional Forwarding

Detection (BFD) for the Pseudowire Virtual Circuit

Connectivity Verification (VCCV)", draft-ietf-pwe3-vccv-bfd-03

(work in progress), February 2009.

Vigoureux, et al.      Expires September 10, 2009              [Page 15]

Internet-Draft        OAM Requirements for MPLS-TP            March 2009

Authors' Addresses

Martin Vigoureux (editor)

Alcatel-Lucent

Email: martin.vigoureux@alcatel-lucent.com

David Ward (editor)

Cisco Systems, Inc.

Email: dward@cisco.com

Malcolm Betts (editor)

Nortel Networks

Email: betts01@nortel.com

Vigoureux, et al.      Expires September 10, 2009              [Page 16]