MPLS-TP OAM Framework and Overview
draft-busi-mpls-tp-oam-framework-01.txt

Status of this Memo

By submitting this Internet-Draft, each author represents that any applicable patent or other IPR claims of which he or she is aware have been or will be disclosed, and any of which he or she becomes aware will be disclosed, in accordance with Section 6 of BCP 79.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups. Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

The list of current Internet-Drafts can be accessed at http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at http://www.ietf.org/shadow.html

This Internet-Draft will expire on June 31, 2009.

Abstract

Multi-Protocol Label Switching (MPLS) Transport Profile (MPLS-TP) is based on a profile of the MPLS and pseudowire (PW) procedures as specified in the MPLS Traffic Engineering (MPLS-TE), pseudowire (PW) and multi-segment PW (MS-PW) architectures complemented with additional Operations, Administration and Maintenance (OAM) procedures for fault, performance and protection-switching management for packet transport applications that do not rely on the presence of a control plane.

This document provides a framework for supporting a comprehensive set of OAM procedures that meets the MPLS-TP OAM requirements [10].

Table of Contents

1. Introduction	3

1.1. Contributing Authors	3

2. Conventions used in this document	3

2.1. Terminology	3

2.2. Definitions	4

3. Functional Components	4

3.1. Maintenance Entity	5

3.2. Maintenance End Points (MEPs)	6

3.3. Maintenance Intermediate Points (MIPs)	7

3.4. Server MEPs	7

4. Reference Model	8

4.1. MPLS-TP Section Monitoring	10

4.2. MPLS-TP LSP End-to-End Monitoring	11

4.3. MPLS-TP LSP Tandem Connection Monitoring	12

4.4. MPLS-TP PW Monitoring	13

4.5. MPLS-TP MS-PW Tandem Connection Monitoring	14

5. OAM Functions for pro-active monitoring	15

5.1. Continuity Check and Connectivity Verification	15

5.1.1. Applications for proactive CC & CV function	17

5.2. Remote Defect Indication	18

5.2.1. Configuration considerations	18

5.2.2. Applications for Remote Defect Indication	18

5.3. Alarm Suppression	19

5.4. Lock Indication	19

5.5. Packet Loss Measurement	19

5.6. Client Signal Fail	19

6. OAM Functions for on-demand monitoring	20

6.1. Continuity Check and Connectivity Verification	20

6.1.1. Configuration considerations	21

6.2. Packet Loss Measurement	21

6.3. Diagnostic Test	21

6.4. Trace routing	21

6.5. Packet Delay Measurement	21

7. OAM Protocols Overview	21

8. Security Considerations	21

9. IANA Considerations	22

10. Acknowledgments	22

11. References	23

11.1. Normative References	23

11.2. Informative References	23

# Introduction

As noted in the architecture for MPLS-TP [8], the overall architecture framework for MPLS-TP is based on a profile of the MPLS-TE and (MS-)PW architectures defined in RFC 3031 [2], RFC 3985 [5] and [6] complemented with additional OAM procedures for fault, performance and protection-switching management for packet transport applications that do not rely on the presence of a control plane.

In line with [11], existing MPLS OAM mechanisms will be used wherever possible and extensions or new OAM mechanisms will be defined only where existing mechanisms are not sufficient to meet the requirements.

The MPLS-TP OAM framework must satisfy the MPLS-TP OAM requirements [10] in a manner that provides for a comprehensive set of OAM procedures. In this regard, it is similar to existing SONET/SDH and OTH OAM mechanisms (e.g. [12]).

## Contributing Authors

Italo Busi, Ben Niven-Jenkins, Annamaria Fulignoli, Enrique Hernandez-Valencia, Lieven Levrau, Dinesh Mohan, Vincenzo Sestito, Nurit Sprecher, Huub van Helvoort, Martin Vigoureux, Yaacov Weingarten

# Conventions used in this document

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in RFC-2119 [1].

## Terminology

LME	LSP Maintenance Entity

LTCME	LSP Tandem Connection Maintenance Entity

ME		Maintenance Entity

MEP	Maintenance End Point

MIP	Maintenance Intermediate Point

PME	PW Maintenance Entity

PTCME	PW Tandem Connection Maintenance Entity

SME	Section Maintenance Entity

## Definitions

MPLS Section: to be added in a future revision of this document.

OAM flow: to be added in a future revision of this document.

Tandem Connection: A tandem connection corresponds to a segment of a path.  This may be either a segment of an LSP (i.e. a sub-path), or one or more segment(s) of a PW.

[Editor’s note – need to review the definition of tandem connection and its relationship with “segment”. If appropriate, we can change the name from “tandem connection” to “segment”.]

# Functional Components

MPLS defines the use of Label Switched Paths (LSPs) and Pseudowires (PWs)[reference the proper RFCs] that are used to connect service end points.  MPLS-TP builds on this framework the need to transport service traffic, based on certain performance and quality measurements.  In order to verify and maintain these performance and quality measurements, we need to use the OAM functionality not only on an end-to-end LSP or PW, but also on sub-paths, defined as Tandem Connections between any two arbitrary points along a path.

MPLS-TP OAM operates in the context of Maintenance Entities (MEs). A Maintenance Entity can be viewed as the association of two (or more) Maintenance End Points (MEPs), see below. The MEPs that form an ME should be configured and managed to limit the OAM responsibilities of an OAM flow within a network or sub-network in the specific layer network that is being monitored and managed. Each OAM flow is associated with a unique ME.

[Editor’s note - Subnetwork generally has a different meaning in IETF than in ITU.  Is what is shown as subnetworks in the above diagram actually transport paths (i.e. G.805 connections) or transport path segments?]

Each MEP resides at the boundaries of the ME that they are part of. An ME may also include a set of zero or more Maintenance Intermediate Points (MIPs), which reside within the Maintenance Entity, between the MEPs.

A MEP is capable of initiating and terminating OAM messages for fault management and performance monitoring.

A MIP is capable of terminating OAM messages but it generates OAM messages only in reaction to received OAM packets.

This functional model defines the relationships between all OAM entities from a maintenance perspective, to allow each Maintenance Entity to monitor and manage the layer network under its responsibility and easily localize problems.

MEPs and MIPs are configured either via the management plane and/or the control plane, and they are associated with a particular Maintenance Entity.

## Maintenance Entity

A Maintenance Entity can be viewed as the association of two (or more) Maintenance End Points (MEPs). An example of an ME with more than two MEPs is a point-to-multipoint ME monitoring a point-to-multipoint LSP. The MEPs that form an ME should be configured and managed to limit the OAM responsibilities of an OAM flow within a network or sub-network, or a transport path or segment, in the specific layer network that is being monitored and managed. Any maintenance point in between MEPs is a Maintenance Intermediate Points (MIP).

A Maintenance Entity may be defined to monitor and manage unidirectional point-to-point or point-to-multipoint connectivity, or bidirectional point-to-point connectivity in an MPLS-TP layer network.

MPLS-TP OAM functions are designed to be applied either on an end-to-end basis, e.g., between the LERs of a given LSP or T-PEs of a given PW, or on a per tandem connection basis, e.g., between any LER/LSR of a given LSP or any T-PE/S-PE of a given PW.

The end points of a tandem connection are MEPs because the tandem connection is by definition a Maintenance Entity.

Therefore, in the context of MPLS-TP LSP or PW Maintenance Entity (defined below) LERs and T-PEs can be MEPs while LSRs and S-PEs may be MIPs. In the case of Tandem Connection Maintenance Entity (defined below), LSRs and S-PEs can be either MEPs or MIPs.

The following properties apply to all MPLS-TP MEs:

- They can be nested but not overlapped.

[Editor’s note – Need to better explain the fact that OAM entities can be nested but not overlapped.

Question is whether we really mean nested.

Proposed rephrased:

OAM entities can be nested but not overlapped, e.g. a ME may cover a sub-path of another ME but all MEPs and MIPs of the sub-path are no longer part of the encompassing ME]

- Each OAM flow is associated with a single Maintenance Entity.

- OAM packets are subject to the same forwarding behavior (e.g. fate share) as the data traffic, but they can be distinguished from the data traffic using the GAL and GE-ACH constructs [9] for LSP and the PW-ACH construct [6] for (MS-)PW.

[Editor’s note – We need to understand whether the solution aspects (e.g. GAL and G-ACH) are in the scope of this draft or not]

## Maintenance End Points (MEPs)

Maintenance End Points (MEPs) are the end points of a pre-configured (through the management or control planes) ME.  MEPs are responsible for activating and controlling all of the OAM functionality for the ME. A MEP may initiate an OAM packet to be transferred to its corresponding MEP, or to an intermediate MIP that is part of the ME.

[Editor’s note – Define MEP source and MEP sink. Are we ok with MEP source/sink terminology or we prefer MEP ingress/egress terminology?]

MEPs prevent OAM packets corresponding to a ME from leaking outside that ME:

- A MEP sink terminates all the OAM packets that it receives corresponding to its ME and does not forward them further along the path. If an unintended connectivity is detected, all traffic on the path is blocked (i.e. all received packets are dropped).

- A MEP source tunnels all the OAM packets that it receives via label stacking. These packets are not processed within the ME as they belong to another ME.

A MEP in a tandem connection is not coincident with the termination of the MPLS-TP transport path (LSP or PW), though it can monitor its connectivity (e.g. count packets). A MEP of an MPLS-TP network transport path is coincident with transport path termination and monitors its connectivity (e.g. count packets).

MPLS-TP MEP notifies a fault indication to the MPLS-TP client layer network.

[Editor’s note – Add some text regarding MEP identification]

## Maintenance Intermediate Points (MIPs)

A Maintenance Intermediate Point (MIP) is a point between the two MEPs in an ME that is capable of reacting to some OAM packets and forwarding all OAM packets while ensuring fate sharing with data plane packets.  A MIP belongs to only one ME.

A MIP does not initiate OAM packets, but may be addressed by OAM packets initiated by one of the MEPs of the ME. A MIP can generate OAM packets only in response to OAM packets that are sent on the ME it belongs to.

[Editor’s note – It is needed to describe about how this is achieved (e.g. TTL expiry). Is this description in the scope of this document?]

MIPs are unaware of any OAM flows running between MEPs or between MEPs and other MIPs. MIPs can only receive and process OAM packets addressed to the MIP itself.

A MIP takes no action on the MPLS-TP transport path.

[Editor’s note – Add some text regarding MIP identification]

## Server MEPs

A server MEP is a MEP of an ME that is defined in a layer network below the MPLS-TP layer network being referenced. A server MEP coincides with either a MIP or a MEP in the client (MPLS-TP) layer network.

For example, a server MEP can be either:

- A termination point of a physical link (e.g. 802.3), an SDH VC or OTH ODU for the MPLS-TP Section layer network, defined in section 4.1. ;

- An MPLS-TP Section MEP for MPLS-TP LSPs, defined in section 4.2. ;

- An MPLS-TP LSP MEP for MPLS-TP PWs, defined in section 4.4. ;

- An MPLS-TP LSP Tandem Connection MEP for higher-level LTCMEs, defined in section 4.3. ;

- An MPLS-TP PW Tandem Connection MEP for higher-level PTCMEs, defined in section 4.5.

The server MEP can run appropriate OAM functions for fault detection, and notifies a fault indication to the MPLS-TP layer network.

# Reference Model

The reference model for MPLS-TP OAM framework builds upon the concept of an ME, and its associated MEPs and MIPs, to support the functional requirements specified in [10].

The following MPLS-TP MEs are specified in this document:

- A Section Maintenance Entity (SME), allowing monitoring and management of MPLS-TP Sections (between MPLS LSRs).

- A LSP Maintenance Entity (LME), allowing monitoring and management of an end-to-end LSP (between LERs).

- A PW Maintenance Entity (PME), allowing monitoring and management of an end-to-end SS/MS-PWs (between T-PEs).

- An LSP Tandem Connection Maintenance Entity (LTCME), allowing monitoring and management of an LSP Tandem Connection between any LER/LSR along the LSP.

- A MS-PW Tandem Connection Maintenance Entity (PTCME), allows monitoring and management of a SS/MS-PW Tandem Connection between any T-PE/S-PE along the (MS-)PW.

[Editor’s note – Need to clarify (in both the framework and requirements) the relationship between TC and segment: e.g. clarify whether a TCME among two segments of a stiching LSP must be supported.]

The MEs specified in this MPLS-TP framework are intended to be compliant with the architecture framework for MS-PWs [7] and MPLS LSPs [2].

Hierarchical LSPs are also supported. In this case, each LSP Tunnel in the hierarchy is an independent layer network that can be monitored end-to-end (from LER to LER) by an LME. Tandem Connection monitoring via LTCME are applicable on each LSP Tunnel in the hierarchy.

Native  |<-------------------- PW15 --------------------->|  Native

Layer   |                                                 |   Layer

Service  |    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |  Service

(AC1)   V    V   LSP   V    V   LSP   V    V   LSP   V    V   (AC2)

+----+   +-+   +----+         +----+   +-+   +----+

+----+        |TPE1|   | |   |SPE3|         |SPEX|   | |   |TPEZ|       +----+

|    |        |    |=========|    |=========|    |=========|    |       |    |

| CE1|--------|........PW1........|...PW3...|........PW5........|-------|CE2 |

|    |        |    |=========|    |=========|    |=========|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

Λ                   Λ         Λ                   Λ

|                   |         |                   |

|<-  Subnetwork A ->|         |<-  Subnetwork Z ->|

∆------------------- PW15  PME -------------------∆

∆---- PW1 PTCME ----∆         ∆---- PW5 PTCME ----∆

∆---------∆                   ∆---------∆

PSN13 LME                     PSNXZ LME

∆---∆ ∆---∆    ∆---------∆    ∆---∆ ∆---∆

Sec12 Sec23       Sec3X       SecXY SecYZ

SME   SME         SME         SME   SME

TPE1: Terminating Provider Edge 1                 SPE2: Switching Provider Edge 3

TPEX: Terminating Provider Edge X                 SPEZ: Switching Provider Edge Z

∆---∆ ME		∆     MEP		====   LSP      .... PW

- Reference Model for the MPLS-TP OAM Framework

Figure 1 depicts a high-level reference model for the MPLS-TP OAM framework. The figure depicts portions of two MPLS-TP enabled subnetworks, Subnetwork A and Subnetwork Z. In Subnetwork A, LSR 1 is adjacent to LSR 2 via the MPLS Section Sec12 and LSR2 is adjacent to LSR3 via the MPLS Section Sec23. Similarly, In Subnetwork Z, LSR X is adjacent to LSR Y via the MPLS Section SecXY and LSR Y is adjacent to LSR Z via the MPLS Section SecYZ. In addition, LSR 3 is adjacent to LSR X via the MPLS Section 3X.

Figure 1 also shows a bi-directional MS-PW (PW15) between AC1 on LSR 1 (TPE1) and AC2 on LSR Z (TPEZ). The MS-PW consists of 3 bi-directional PW Segments: 1) PW Segment 1 (PW1) between LSR 1 (TPE1) and LSR 3 (SPE3) via the bi-directional PSN13 LSP, 2) PW Segment 3 (PW3) between LSR 3 (SPE3) and LSR X (SPEX), and 3) PW Segment 5 (PW5) between LSR X (SPEX) and LSR Z (TPEZ) via the bi-directional PSNXZ LSP.

The MPLS-TP OAM procedures that apply to an instance of a given ME are expected to operate independently from procedures on other instances of the same ME and certainly of other MEs. Yet, this does not preclude that multiple MEs may be affected simultaneously by the same network condition, for example, a fiber cut event.

Note that there are no constrains imposed by this OAM framework on the number, or type, of MEs that may be instantiated a particular monitoring point. In particular, when looking at Figure 1, it should be possible to configure one or more MEs from the same MEP if each of the MEs share the same monitoring point.

The subsections below define the MEs specified in this MPLS-TP OAM architecture framework document. Unless otherwise stated, all references to subnetworks, LSRs, MPLS Sections, LSP, pseudowires and MEs in this Section are made in relation to those shown in Figure 1.

## MPLS-TP Section Monitoring

An MPLS-TP Section ME (SME) is an MPLS-TP maintenance entity intended to monitor the forwarding behaviour of an MPLS Section as defined in [9]. An SME may be configured on any MPLS section. SME OAM packets fate share with the user data packets sent over the monitored MPLS Section.

An SME is intended to be deployed for applications where it is preferable to monitor the link between the topologically adjacent MPLS (and MPLS-TP enabled) LSRs rather than monitoring the individual LSP or PW segments traversing the MPLS Section. A representative application is collecting link-level PM statistics at the node-to-node interfaces (NNI) in MPLS-TP sub-network domains.

[EJH: Is this language sufficient to cover the case where the link is implemented via ETH LAG/?]

|<-------------------- PW15 --------------------->|

|                                                 |

|    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |

V    V   LSP   V    V   LSP   V    V   LSP   V    V

+----+   +-+   +----+         +----+   +-+   +----+

+----+        |TPE1|   | |   |SPE3|         |SPEX|   | |   |TPEZ|       +----+

|    |  AC1   |    |=========|    |=========|    |=========|    |  AC2  |    |

| CE1|--------|........PW1........|...PW3...|........PW5........|-------|CE2 |

|    |        |    |=========|    |=========|    |=========|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

∆--∆  ∆--∆     ∆--------∆     ∆--∆  ∆--∆

Sec12 SME Sec23 SME Sec3X SME SecXY SME SecYZ SME

- Reference Example of MPLS-TP Section MEs (SME)

Figure 2 shows 5 Section MEs configured in the path between AC1 and AC2: 1) Sec12 ME associated with the MPLS Section between LSR 1 and LSR 2, 2) Sec23 ME associated with the MPLS Section between LSR 2 and LSR 3, 3) Sec3X ME associated with the MPLS Section between LSR 3 and LSR X, 4) SecXY ME associated with the MPLS Section between LSR X and LSR Y, and 5) SecYZ ME associated with the MPLS Section between LSR Y and LSR Z.

## MPLS-TP LSP End-to-End Monitoring

An MPLS-TP LSP ME (LME) is an MPLS-TP maintenance entity intended to monitor the forwarding behaviour of an end-to-end LSP between two (e.g., a point-to-point LSP) or more (e.g., a point-to-multipoint LSP) LERs. An LME may be configured on any MPLS LSP. LME OAM packets fate share with user data packets sent over the monitored MPLS LSP.

An LME is intended to be deployed in scenarios where it is desirable to monitor the forwarding behaviour of an entire LSP between a pair of MPLS LERs, rather than, say, monitoring individual PWs. A representative application is collecting PM statistics of PSN LSP that is being used to provide a “tunnelling services” for a number of other LSPs.

|<-------------------- PW15 --------------------->|

|                                                 |

|    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |

V    V   LSP   V    V   LSP   V    V   LSP   V    V

+----+   +-+   +----+         +----+   +-+   +----+

+----+        |TPE1|   | |   |SPE3|         |SPEX|   | |   |TPEZ|       +----+

|    |  AC1   |    |=========|    |=========|    |=========|    |  AC2  |    |

| CE1|--------|........PW1........|...PW3...|........PW5........|-------|CE2 |

|    |        |    |=========|    |=========|    |=========|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

∆---------∆                   ∆---------∆

PSN13 LME                     PSNXZ LME

- Examples of MPLS-TP LSP MEs (LME)

Figure 3 depicts 2 LMEs configured in the path between AC1 and AC2: 1) the PSN13 LME between LER 1 and LER 3, and 2) the PSNXZ LME between LER X and LER Y. Note that the presence of a PSN3X ME in such a configuration is optional, hence, not precluded by this framework. For instance, the SPs may prefer to monitor the MPLS Section between the two LSRs rather than the individual LSPs.

## MPLS-TP LSP Tandem Connection Monitoring

An MPLS-TP LSP Tandem Connection Monitoring ME (LTCME) is an MPLS-TP maintenance entity intended to monitor the forwarding behaviour of an LSP Segment between a given pair of LSRs. Multiple LTCMEs MAY BE configured on any LSP Segment. The LSR may or may not be immediately adjacent at the MPLS layer. LTCME OAM packets fate share with the user data packets sent over the monitored LSP segment.

A LTCME can be defined between the following entities:

  - LER and any LSR of a given LSP.

  - Any two LSRs of a given LSP.

An LTCME is intended to be deployed in scenarios where it is preferable to monitor the behaviour of a segment of an LSP rather than the entire LSP itself. A representative application is when there is a need to monitor a segment of an LSP that extends beyond the administrative boundaries of an MPLS-TP enabled administrative domain.

Note that LTCMEs are equally applicable to hierarchical LSPs.

|<--------------------- PW15 -------------------->|

|                                                 |

|    |<--------------PSN1Z LSP-------------->|    |

|    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |

V    V  S-LSP  V    V  S-LSP  V    V  S-LSP  V    V

+----+   +-+   +----+         +----+   +-+   +----+

+----+        | PE1|   | |   | PB3|         | PBX|   | |   | PEZ|       +----+

|    |  AC1   |    |=======================================|    |  AC2  |    |

| CE1|--------|......................PW15.......................|-------|CE2 |

|    |        |    |=======================================|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

∆---------∆                   ∆---------∆

PSN13 LTCME                   PSNXZ LTCME

PB: Provider Border LSR

- MPLS-TP LSP Tandem Conection Monitoring ME (LTCME)

Figure 4 depicts a variation of the reference model in Figure 1 where there is an end-to-end PSN LSP (PSN1Z LSP) between PE1 and PEZ. PSN1Z LSP consists of, at least, three stitched LSP Segments: PSN13, PSN3X and PSNXZ. In this scenario there are two separate LTCMEs configured to monitor the forwarding behaviour of the PSN1Z LSP: 1) a LTCME monitoring the PSN13 LSP Segment on Subnetwork 123 (PSN13 LTCME), and 2) a LTCME monitoring the PSNXZ LSP Segment on Subnetwork XYZ (PSNXZ LTCME).

## MPLS-TP PW Monitoring

An MPLS-TP PW ME (PME) is an MPLS-TP maintenance entity intended to monitor the end-to-end forwarding behaviour of a SS-PW or MS-PW between a pair of T-PEs. A PME MAY be configured on any SS-PW or MS-PW. PME OAM packets fate share with the user data packets sent over the monitored PW.

A PME is intended to be deployed in scenarios where it is desirable to monitor the forwarding behaviour of an entire PW between a pair of MPLS-TP enabled T-PEs rather than monitoring the LSP aggregating multiple PWs between PEs. A representative application is on either SS-PW or MS-PW used to emulate traffic for which an SLA with QoS commitments may apply (e.g., an emulated DS1/E1 or the emulated CBR connection of an ATM VCC/VPC).

|<-------------------- PW15 --------------------->|

|                                                 |

|    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |

V    V   LSP   V    V   LSP   V    V   LSP   V    V

+----+   +-+   +----+         +----+   +-+   +----+

+----+        |TPE1|   | |   |SPE3|         |SPEX|   | |   |TPEZ|       +----+

|    |  AC1   |    |=========|    |=========|    |=========|    |  AC2  |    |

| CE1|--------|........PW1........|...PW3...|........PW5........|-------|CE2 |

|    |        |    |=========|    |=========|    |=========|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

∆---------------------PW15 PME--------------------∆

- MPLS-TP PW ME (PME)

Figure 5 depicts a MS-PW (PW15) consisting of three segments: PW1, PW3 and PW5 and its associated end-to-end PME (PW15 PME).

## MPLS-TP MS-PW Tandem Connection Monitoring

An MPLS-TP MS-PW Tandem Connection Monitoring ME (PTCME) is an MPLS-TP maintenance entity intended to monitor the forwarding behaviour of one or more MS-PW segments between a given pair of PEs. Multiple PTCMEs MAY be configured on any sub-path of a MS-PW. The PEs may or may not be immediately adjacent at the MS-PW layer. PTCME OAM packets fate share with the user data packets sent over the monitored MS-PW Segment.

- A PTCME can be defined between the following entities:

  - T-PE and any S-PE of a given MS-PW

  - Any two S-PEs of a given MS-PW. It can span several PW segments.

A PTCME is intended to be deployed in scenarios where it is preferable to monitor the behaviour of a segment of a MS-PW rather than the entire end-to-end PW itself. A representative application is to collect PM statistics for the MS-PW Segment within a given network domain of an inter-domain PW.

|<-------------------- PW15 --------------------->|

|                                                 |

|    |<-PSN13->|    |<-PSN3X->|    |<-PSNXZ->|    |

V    V   LSP   V    V   LSP   V    V   LSP   V    V

+----+   +-+   +----+         +----+   +-+   +----+

+----+        |TPE1|   | |   |SPE3|         |SPEX|   | |   |TPEZ|       +----+

|    |  AC1   |    |=========|    |=========|    |=========|    |  AC2  |    |

| CE1|--------|........PW1........|...PW3...|........PW5........|-------|CE2 |

|    |        |    |=========|    |=========|    |=========|    |       |    |

+----+        | 1  |   |2|   | 3  |         | X  |   |Y|   | Z  |       +----+

+----+   +-+   +----+         +----+   +-+   +----+

∆---- PW1 PTCME ----∆         ∆---- PW5 PTCME ----∆

- MPLS-TP MS-PW Tandem Connection Monitoring (PTCME)

Figure 6 depicts the same MS-PW (PW15) between AC1 and AC2 as in Figure 5. In this scenario there are two separate PTCMEs configured to monitor the forwarding behaviour of PW15: 1) a PTCME monitoring the PW1 MS-PW Segment on Subnetwork 123 (PW1 PTCME), and 2) a PTCME monitoring the PW4 MS-PW Segment on Subnetwork XYZ with (PW5 PTCME).

[Editor’s note - it could be underlined that this two PTCMEs can be co-exist together with the PTCME of the previous scenario (figure 6). i.e to underline that on a monitoring point more than one MEP can be configured at the same time]

# OAM Functions for pro-active monitoring

## Continuity Check and Connectivity Verification

Proactive Continuity and Connectivity Verification (CC & CV) function is used to detect loss of continuity (LOC), unexpected connectivity between two MEs (e.g. mismerging or misconnection) as well as unexpected connectivity within the ME with an unexpected MEP.

Proactive CC & CV is based upon the generation of OAM pro-active CC/CV packets, carrying a unique ME identifier, at a regular configurable timing rate and the detection of LOC when these packets do not arrive. If the received ME identifier does not match the expected ME identifier, a connectivity defect has occurred. The default CC/CV transmission periods are application dependent (see section 5.1.1. )

[Editor’s Note – CC packets should be transmitted with the "minimum loss-probability PHB". Is PHB configurable? If not, how the PHB for CC is decided?]

For statically provisioned connections, the transmission period and the ME identifier are statically configured at both MEPs. For dynamically established connections, the transmission period and the ME identifier are signaled via the control plane.

In a bidirectional point-to-point connection, when a MEP is enabled to generate pro-active CC/CV packets with a configured transmission period, it also expects to receive pro-active CC/CV packets from its peer MEP with the same transmission period. In a unidirectional connection (point-to-point or point-to-multipoint), only the source MEP is enabled to generate packets with CC/CV information. This MEP does not expect to receive any packets with CC/CV information from its peer MEPs in the ME.

MIPs as well as intermediate nodes not supporting MPLS-TP OAM are transparent to the pro-active CC/CV information and forward pro-active CC/CV packets as regular data packets.

When CC & CV is enabled, a MEP periodically transmits pro-active CC/CV packets with frequency of the configured transmission period.

When a MEP enabled to receive pro-active CC/CV packets

When CC & CV is enabled, a MEP detects loss of continuity (LOC) defect with a peer MEP when it receives no pro-active CC/CV packets from the peer MEP within the interval equal to 3.5 times the transmission period.

When a pro-active CC/CV packet is received, a MEP is able to detect a mis-connectivity defect (e.g. mismerge or misconnection) with another ME when the received packet carries an incorrect ME identifier

If pro-active CC/CV packets are received with a transmission period different than expected, CC/CV period mis-configuration defect is detected.

[Editor’s note – We need to understand whether a mechanism for auto-negotiate the actual transmission period such that in case of period mis-configuration the two MEPs converge on the slower speed is required. It is anyway important to report to the operator the fact that the negotiated speed mismatches the configured one. Do we need to add some text to capture the capability to auto-negotiate the rate between MEPs?]

[Editor’s note – Need to add the defect clearing conditions]

A receiving MEP notifies the equipment fault management process when it detects the above defect conditions.

If a MEP detects an unexpected connectivity it MUST block all the traffic (including also the user data packets) that it receives from the misconnected connection.

It is worth noticing that the OAM requirements I-D recommends [10] that CC-CV proactive monitoring is enabled on every ME in order to reliably detect connectivity defects.

However, CC-CV proactive monitoring can be disabled by an operator on a ME. In this case a dLOC can be a connectivity problem (e.g. a misconnection with a connection where CC-CV proactive monitoring is not enabled) and not necessarily a continuity problem, with a consequent wrong traffic delivering.

For these reasons, the traffic block consequent action SHOULD be applied even when a LOC condition occurs.

The activation of the traffic block consequent action should be configurable (i.e. it should be possible to enable/disable the consequent action) in case of LOC condition; that in order to enable/disable the proactive CC-CV monitoring on a ME in a not traffic affecting way.

### Applications for proactive CC & CV function

CC & CV is applicable for fault management, performance monitoring, or protection switching applications.

- Fault Management: default transmission period is 1s (i.e. transmission rate of 1 packet/second)

- Performance Monitoring: default transmission period is 100ms (i.e. transmission rate of 10 packets/second)

- Protection Switching: in order to achieve sub-50ms recovery time the default transmission period is 3.33ms (i.e. transmission rate of 300 packets/second) although a transmission period of 10ms can also be used. In some cases, when a slower recovery time is acceptable, it is also possible to relax the transmission period.

[Editor’s note – We can turn this into something more formulaic. Given a desired switching time of Xms, and a hardware switching time of Yms, and an OAM message propagation time of Zms, the transmission period should be set to no greater than t = (X-Y-Z)/n assuming that switchover will not be triggered until n protection switching messages have failed to be received.]

## Remote Defect Indication

The Remote Defect Indication (RDI) is an indicator that is transmitted by a MEP to communicate to its peer MEPs that a signal fail condition exists.  RDI is only used for bidirectional connections and is associated with proactive CC & CV packet generation.

[Editor’s note – Add more specific information about the signal fail conditions reported by RDI.]

A MEP that has identified a signal fail related defect should include the RDI in all pro-active CC/CV packets that it generates for the duration of the defect condition existence.  A MEP that receives the packets with the RDI information should determine that its peer MEP has encountered a defect condition associated with a signal fail.  MIPs should be transparent to the RDI indicator and should forward packets that include the indicator, i.e. the MIP should not perform any actions nor examine the indicator.

When the signal condition clears, the MEP should clear the RDI indicator from subsequent transmission of pro-active CC/CV packets.  Peer MEP, that have set the RDI condition, should clear the condition upon reception of a packet from the source MEP with the RDI indicator cleared.

### Configuration considerations

In order to support RDI indication, the RDI transmission rate and PHB of the MEP should be configured as part of the CC & CV configuration.

### Applications for Remote Defect Indication

RDI is applicable for the following applications:

- Single-ended fault management – A receiving MEP detects the RDI defect condition, which when correlated with other defect conditions in the receiving MEP may become a fault case.

- Contribution to far-end performance monitoring – The indication of the far-end defect condition is used as input to the performance monitoring process.

## Alarm Suppression

Alarm Indication Signal function (AIS) is used to suppress alarms following detection of defect conditions at the server (sub) layer.

- Packets with AIS information can be issued at a MEP, including a Server MEP, upon detecting defect conditions.

A server MEP is responsible for notifying the MPLS-TP layer network MEP upon fault detection in the server layer network to which the server MEP is associated.

Only Server MEPs can issue MPLS-TP packets with AIS information. Upon detection of a signal fail condition the Server MEP can immediately start transmitting packets with AIS information periodically. A Server MEP continues to transmit periodic packets with AIS information until the signal fail condition is cleared.

Upon receiving a packet with AIS information a MEP detects an AIS defect condition and suppresses loss of continuity alarms associated with all its peer MEPs.  A MEP resumes loss of continuity alarm generation upon detecting loss of continuity defect conditions in the absence of AIS condition.

Specific configuration information required by a MEP to support AIS transmission is the following:

- PHB – identifies the per-hop behaviour of packet with AIS information.

A MIP is transparent to packets with AIS information and therefore does not require any information to support AIS functionality.

## Lock Indication

To be incorporated in a future revision of this document

## Packet Loss Measurement

To be incorporated in a future revision of this document

## Client Signal Fail

To be incorporated in a future revision of this document

# OAM Functions for on-demand monitoring

## Continuity Check and Connectivity Verification

In order to preserve network resources, e.g. bandwidth, processing time at switches, it may be preferable to not use continual pro-active CC & CV.  In order to perform fault management functions network management may invoke periodic on-demand bursts of on-demand CC/CV packets.  Use of on-demand CC & CV is dependent on the existence of a bi-directional connection ME.

An additional use of on-demand CC & CV would be to detect and locate a problem of connectivity when a problem is suspected or known based on other tools.  In this case the functionality will be triggered by the network management in response to a status signal or alarm indication.

On-demand CC & CV is based upon generation of on-demand CC/CV packets that should uniquely identify the ME that is being checked.  The on-demand functionality may be used to check either an entire ME (end-to-end) or between a MEP to a specific MIP.

On-demand CC & CV may generate a one-time burst of on-demand CC/CV packets, or be used to invoke periodic, non-continuous, bursts of on-demand CC/CV packets.  The number of packets generated in each burst is configurable at the MEPs, and should take into account normal packet-loss conditions.

When invoking a periodic check of the ME, the source MEP should issue a burst of on-demand CC/CV packets that uniquely identifies the ME being verified.  The number of packets and their transmission rate should be pre-configured and known to both the source MEP and the target MEP or MIP.  The source MEP should use the TTL field to indicate the number of hops necessary, when targeting a MIP and use the default value when performing an end-to-end check [IB => This is quite generic for addressing packets to MIPs and MEPs so it is better to move this text in section 2].  The target MEP/MIP shall return a reply on-demand CC/CV packet for each packet received.  If the expected number of on-demand CC/CV reply packets is not received at source MEP, a LOC state is detected.

[Editor’s note – We need to add some text for the usage of CC&CV with different packet sizes, e.g. to discover MTU problems. Question for requirements: MTU problems should be only detected in the on-demand? Do we wish to have it pro-active?]

When a connectivity problem is detected (e.g. via a pro-active CC&CV OAM tool), on demand CC&CV tool can be used to check the path.  The series should check CC&CV from MEP to peer MEP on the path, and if a fault is discovered, by lack of response, then additional checks may be performed to each of the intermediate MIP to locate the fault.

### Configuration considerations

For on-demand CC & CV the MEP should support configuration of number of packets to be transmitted/received in each burst of transmissions and the transmission rate should be either pre-configured or negotiated between the different nodes.

In addition, when the CC & CV packet is checking connectivity toward a target MIP, the number of hops to reach the target MIP should be configured.

The PHB of the on-demand CC/CV packets should be configured as well.

[Editor’s note – We need to be better define the reason for such configuration]

## Packet Loss Measurement

To be incorporated in a future revision of this document

## Diagnostic Test

To be incorporated in a future revision of this document

## Trace routing

To be incorporated in a future revision of this document

## Packet Delay Measurement

To be incorporated in a future revision of this document

# OAM Protocols Overview

To be incorporated in a future revision of this document

# Security Considerations

To be incorporated in a future revision of this document

# IANA Considerations

To be incorporated in a future revision of this document

# Acknowledgments

The authors would like to thank all members of the teams (the Joint Working Team, the MPLS Interoperability Design Team in IETF and the T-MPLS Ad Hoc Group in ITU-T) involved in the definition and specification of MPLS Transport Profile.

This document was prepared using 2-Word-v2.0.template.dot.

# References

## Normative References

- Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997

- Rosen, E., Viswanathan, A., Callon, R., "Multiprotocol Label Switching Architecture", RFC 3031, January 2001

- Rosen, E., et al., "MPLS Label Stack Encoding", RFC 3032, January 2001

- Agarwal, P., Akyol, B., "Time To Live (TTL) Processing in Multi-Protocol Label Switching (MPLS) Networks", RFC 3443, January 2003

- Bryant, S., Pate, P., "Pseudo Wire Emulation Edge-to-Edge (PWE3) Architecture", RFC 3985, March 2005

- Nadeau, T., Pignataro, S., "Pseudowire Virtual Circuit Connectivity Verification (VCCV): A Control Channel for Pseudowires", RFC 5085, December 2007

- Bocci, M., Bryant, S., "An Architecture for Multi-Segment Pseudo Wire Emulation Edge-to-Edge", draft-ietf-pwe3-ms-pw-arch-05 (work in progress), September 2008

- Bocci, M., et al., " A Framework for MPLS in Transport Networks", draft-ietf-mpls-tp-framework-00 (work in progress), November 2008

- Vigoureux, M., Bocci, M., Swallow, G., Ward, D., Aggarwal, R., " MPLS Generic Associated Channel ", draft-ietf-mpls-tp-gach-gal-01 (work in progress), January 2009

## Informative References

- Vigoureux, M., Betts, M., Ward, D., "Requirements for OAM in MPLS Transport Networks", draft-ietf-mpls-tp-oam-requirements-00 (work in progress), November 2008

- Sprecher, N., Nadeau, T., van Helvoort, H., Weingarten, Y., " MPLS-TP OAM Analysis", draft-sprecher-mpls-tp-oam-analysis-02 (work in progress), September 2008

- ITU-T Recommendation G.707/Y.1322 (01/07), " Network node interface for the synchronous digital hierarchy (SDH)", 2007

Authors' Addresses

Italo Busi (Editor)

Alcatel-Lucent

Email: Italo.Busi@alcatel-lucent.it

Ben Niven-Jenkins (Editor)

BT

Email: benjamin.niven-jenkins@bt.com

Contributing Authors' Addresses

Annamaria Fulignoli

Ericsson

Email: annamaria.fulignoli@ericsson.com

Enrique Hernandez-Valencia

Alcatel-Lucent

Email: enrique@alcatel-lucent.com

Lieven Levrau

Alcatel-Lucent

Email: llevrau@alcatel-lucent.com

Dinesh Mohan

Nortel

Email: mohand@nortel.com

Vincenzo Sestito

Alcatel-Lucent

Email: vincenzo.sestito@alcatel-lucent.it

Nurit Sprecher

Nokia Siemens Networks

Email: nurit.sprecher@nsn.com

Huub van Helvoort

Huawei Technologies

Email: hhelvoort@huawei.com

Martin Vigoureux

Alcatel-Lucent

Email: martin.vigoureux@alcatel-lucent.fr

Yaacov Weingarten

Nokia Siemens Networks

Email: yaacov.weingarten@nsn.com

Full Copyright Statement

Copyright (C) The IETF Trust (2009).

This document is subject to the rights, licenses and restrictions contained in BCP 78, and except as set forth therein, the authors retain all their rights.

This document and the information contained herein are provided on an "AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY, THE IETF TRUST AND THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any Intellectual Property Rights or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; nor does it represent that it has made any independent effort to identify any such rights.  Information on the procedures with respect to rights in RFC documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the IETF on-line IPR repository at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights that may cover technology that may be required to implement this standard.  Please address the information to the IETF at ietf-ipr@ietf.org.