rtgwg                                                           N. Kumar

Internet-Draft                                              C. Pignataro

Intended status: Informational                                  D. Kumar

Expires: September 13, 2016                          Cisco Systems, Inc.

G. Mirsky

Ericsson

M. Chen

Huawei Technologies

E. Nordmark

Arista Networks

S. Pallagatti

Juniper Networks

D. Mozes

Mellanox Technologies Ltd

March 12, 2016

Overlay OAM Requirements

draft-ooamdt-rtgwg-ooam-requirement-00

Abstract

This document describes a list of functional requirements for

Operations Administration and Maintenance (OAM) in various Overlay

and Service networks like Service Function Chaining (SFC), Bit Index

Explicit Replication (BIER), Network Virtualization over L3 (NVO3).

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at http://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on September 13, 2016.

Kumar, et al.          Expires September 13, 2016               [Page 1]

Internet-Draft          Overlay OAM Requirements              March 2016

Copyright Notice

Copyright (c) 2016 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents

(http://trustee.ietf.org/license-info) in effect on the date of

publication of this document.  Please review these documents

carefully, as they describe your rights and restrictions with respect

to this document.  Code Components extracted from this document must

include Simplified BSD License text as described in Section 4.e of

the Trust Legal Provisions and are provided without warranty as

described in the Simplified BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   2

2.  Requirements notation . . . . . . . . . . . . . . . . . . . .   3

3.  Terminology . . . . . . . . . . . . . . . . . . . . . . . . .   3

4.  Detailed Requirement list . . . . . . . . . . . . . . . . . .   4

4.1.  Fault Management  . . . . . . . . . . . . . . . . . . . .   4

4.1.1.  Pro-active Fault Management . . . . . . . . . . . . .   4

4.1.2.  On-demand Fault Management  . . . . . . . . . . . . .   5

4.2.  Performance Management  . . . . . . . . . . . . . . . . .   5

4.3.  Alarm Indication Suppression  . . . . . . . . . . . . . .   6

4.4.  Overlay Network Resiliency  . . . . . . . . . . . . . . .   6

5.  IANA Considerations . . . . . . . . . . . . . . . . . . . . .   6

6.  Security Considerations . . . . . . . . . . . . . . . . . . .   6

7.  Acknowledgement . . . . . . . . . . . . . . . . . . . . . . .   6

8.  References  . . . . . . . . . . . . . . . . . . . . . . . . .   6

8.1.  Normative References  . . . . . . . . . . . . . . . . . .   6

8.2.  Informative References  . . . . . . . . . . . . . . . . .   8

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .   8

1.  Introduction

We have witnessed, participated in design of new paradigms in the networking that are aimed to address network virtualization, service function chaining, and multicast services. New paradigms require new architectural concepts, principles and components.

[RFC7365] defines a

framework for Data Center Network Virtualization over Layer 3 (NVO3).

[RFC7665] describes the architecture for creating and maintaining

Service Function Chains (SFCs) in a network.

[I-D.ietf-bier-architecture] defines a stateless multicast

architecture for optimal multicast packet forwarding using "Bit Index

Explicit Replication" (BIER).  These frameworks are defined in a

flexible manner that they are transport agnostic and may

be deployed on various underlay networks, such as IPv4, IPv6 and/or MPLS

.

Kumar, et al.          Expires September 13, 2016               [Page 2]

Internet-Draft          Overlay OAM Requirements              March 2016

New architectural concepts and principles have been combined into new network layers with distinct encapsulation headers.  For example, [I-D.ietf-sfc-nsh] defines

an encapsulation header as Network Service Header (NSH) to realize

Service Function Path.  While [RFC7348] (VxLAN) and [RFC7637] (NVGRE)

are different encapsulation header proposed for NVO3,

[I-D.ietf-nvo3-vxlan-gpe] extends VxLAN further to be used for

Service Function Chain (SFC).  Similarly,

[I-D.ietf-bier-mpls-encapsulation] defines the BIER encapsulation

header over MPLS network and [I-D.xu-bier-encapsulation] describes

the BIER encapsulation header over IP network.

Introduction of the new Overlay networks

sets forth new Operations, Administration and Maintenance (OAM) requirements that can be addressed by enhancement of existing and development of OAM new protocols.

For example, [I-D.ietf-sfc-oam-framework] defines the

framework for SFC OAM, [I-D.nordmark-nvo3-transcending-traceroute]

proposes a way to perform traceroute in NVO3 networks and

[I-D.kumarzheng-bier-ping] proposes on-demand connectivity

verification and fault isolation procedure (Ping and Trace) on BIER

network.

The goal of this document is to identify and list the

OAM requirements commonly applicable

to new Overlay networks which can further be used to analyze the

existing Overlay OAM tools. The identified gaps can be further addressed

through enhancing existing OAM tools and, if necessary, constructing new OAM tools,

that can be used as a common unified OAM toolset to support and

perform various OAM functions including proactive and on-demand

path monitoring and service validation on the new Overlay

networks.

2.  Requirements notation

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

3.  Terminology

ECMP: Equal Cost Multipath

UCMP: Unequal Cost Multipath

SFC: Service Function Chaining

BIER: Bit Index Explicit Replication

NVO3: Network Virtualization over L3

OAM: Operation, Administration and Maintenance

MPLS: Multiprotocol Label Switching

VxLAN: Virtual Extensible Local Area Network

Kumar, et al.          Expires September 13, 2016               [Page 3]

Internet-Draft          Overlay OAM Requirements              March 2016

4.  Detailed Requirement List

This section lists the OAM requirements for different Overlay networks.

The below listed requirements MUST be supported with any underlay

transport network:

REQ#1:  The listed requirements MUST be supported with any type of

transport layer over which the overlay network can be

realized

REQ#2:  It MUST be possible to initialize Overlay OAM session from

any node in the overlay network.

REQ#3:  It SHOULD be possible to initialize an Overlay OAM session

from a centralized controller.

REQ#4:  Overlay OAM MUST support proactive and on-demand OAM

monitoring and measurement methods.

REQ#5:  Overlay OAM MUST support unidirectional OAM methods, both

continuity check and performance measurement.

REQ#6:  Overlay OAM packets SHOULD be fate sharing, i.e. in-band, with the monitored

traffic, i.e. follow exactly the same path as data plane

traffic, in forward direction, i.e. from ingress toward

egress end point(s) of the OAM test session.

REQ#7:  Overlay OAM MUST support bi-directional OAM methods.  Such

OAM methods MAY combine in-band monitoring or measurement in

forward direction and out-of-band notification in the

reverse direction, i.e. from egress to ingress end point of

the OAM test session.

4.1.  Fault Management

4.1.1.  Pro-active Fault Management

Availability, not as performance metric, is understood as ability to

reach the node, i.e. the fact that path between ingress and egress

does exist.  Such OAM mechanism also referred as Continuity Check.

REQ#8:  Overlay OAM MUST support pro-active monitoring of any virtual

node availability in the given overlay network.

REQ#9:  Overlay OAM MUST support Reverse Defect Indication (RDI)

notification by egress to the ingress, i.e. source of

continuity checking.

Kumar, et al.          Expires September 13, 2016               [Page 4]

Internet-Draft          Overlay OAM Requirements              March 2016

REQ#10: Overlay OAM MUST support connectivity verification.

Definition of mis-connectivity defect entry and exit

criteria are outside the scope of this document.

4.1.2.  On-demand Fault Management

REQ#11: Overlay OAM MUST support fault localization of Loss of

Continuity check.

REQ#12: Overlay OAM MUST support tracing path in overlay network

through the virtual nodes.

REQ#13: Overlay OAM MAY support verification of the mapping between

its data plane state and client layer services.

REQ#14: Overlay OAM MUST have the ability to discover and exercise

equal cost multipath (ECMP) paths in its transport network.

REQ#15: Overlay OAM MUST be able to trigger on-demand FM with

responses being directed towards initiator of such proxy

request.

4.2.  Performance Management

REQ#16:  Overlay OAM MUST support active one-way packet delay

measurement.

REQ#17:  Overlay OAM MUST support passive one-way packet delay

measurement.

REQ#18:  Overlay OAM MUST support active two-way packet delay

measurement.

REQ#19:  Overlay OAM MUST support packet delay variation measurement.

REQ#20:  Overlay OAM MUST support active end to end packet loss

measurement.

REQ#21:  Overlay OAM MUST support passive end to end packet loss

measurement.

REQ#22:  Overlay OAM SHOULD support active per-segment packet delay

measurement.

REQ#23:  Overlay OAM SHOULD support passive per-segment packet delay

measurement.

Kumar, et al.          Expires September 13, 2016               [Page 5]

Internet-Draft          Overlay OAM Requirements              March 2016

REQ#24:  Overlay OAM SHOULD support active per-segment packet loss

measurement.

REQ#25:  Overlay OAM SHOULD support passive per-segment packet loss

measurement.

REQ#26:  Overlay OAM MUST support delivered packet throughput

measurement.

4.3.  Alarm Indication Suppression

REQ#27: Overlay OAM MUST support defect notification mechanism, like

Alarm Indication Signal.

REQ#28: Any virtual node in the given overlay network MAY originate a

defect notification addressed to any node in that network.

4.4.  Overlay Network Resiliency

REQ#29: Overlay OAM MUST support methods to enable survivability of

an overlay network.  These recovery methods MAY use

protection switching and restoration.

5.  IANA Considerations

This document does not propose any IANA consideration.

6.  Security Considerations

This document list the OAM requirement for various Overlay network

and does not raise any security considerations.

7.  Acknowledgement

TBD

8.  References

8.1.  Normative References

[I-D.ietf-bier-architecture]

Wijnands, I., Rosen, E., Dolganow, A., P, T., and S.

Aldrin, "Multicast using Bit Index Explicit Replication",

draft-ietf-bier-architecture-03 (work in progress),

January 2016.

Kumar, et al.          Expires September 13, 2016               [Page 6]

Internet-Draft          Overlay OAM Requirements              March 2016

[I-D.ietf-bier-mpls-encapsulation]

Wijnands, I., Rosen, E., Dolganow, A., Tantsura, J., and

S. Aldrin, "Encapsulation for Bit Index Explicit

Replication in MPLS Networks", draft-ietf-bier-mpls-

encapsulation-03 (work in progress), February 2016.

[I-D.ietf-nvo3-vxlan-gpe]

Quinn, P., Manur, R., Kreeger, L., Lewis, D., Maino, F.,

Smith, M., Agarwal, P., Yong, L., Xu, X., Elzur, U., Garg,

P., and D. Melman, "Generic Protocol Extension for VXLAN",

draft-ietf-nvo3-vxlan-gpe-01 (work in progress), November

2015.

[I-D.ietf-sfc-nsh]

Quinn, P. and U. Elzur, "Network Service Header", draft-

ietf-sfc-nsh-02 (work in progress), January 2016.

[I-D.ietf-sfc-oam-framework]

Aldrin, S., Krishnan, R., Akiya, N., Pignataro, C., and A.

Ghanwani, "Service Function Chaining Operation,

Administration and Maintenance Framework", draft-ietf-sfc-

oam-framework-01 (work in progress), February 2016.

[I-D.kumarzheng-bier-ping]

Kumar, N., Pignataro, C., Akiya, N., Zheng, L., Chen, M.,

and G. Mirsky, "BIER Ping and Trace", draft-kumarzheng-

bier-ping-02 (work in progress), December 2015.

[I-D.nordmark-nvo3-transcending-traceroute]

Nordmark, E., Appanna, C., and A. Lo, "Layer-Transcending

Traceroute for Overlay Networks like VXLAN", draft-

nordmark-nvo3-transcending-traceroute-01 (work in

progress), October 2015.

[I-D.xu-bier-encapsulation]

Xu, X., Somasundaram, S., Jacquenet, C., and R. Raszuk,

"BIER Encapsulation", draft-xu-bier-encapsulation-03 (work

in progress), October 2015.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119,

DOI 10.17487/RFC2119, March 1997,

<http://www.rfc-editor.org/info/rfc2119>.

Kumar, et al.          Expires September 13, 2016               [Page 7]

Internet-Draft          Overlay OAM Requirements              March 2016

[RFC7348]  Mahalingam, M., Dutt, D., Duda, K., Agarwal, P., Kreeger,

L., Sridhar, T., Bursell, M., and C. Wright, "Virtual

eXtensible Local Area Network (VXLAN): A Framework for

Overlaying Virtualized Layer 2 Networks over Layer 3

Networks", RFC 7348, DOI 10.17487/RFC7348, August 2014,

<http://www.rfc-editor.org/info/rfc7348>.

[RFC7365]  Lasserre, M., Balus, F., Morin, T., Bitar, N., and Y.

Rekhter, "Framework for Data Center (DC) Network

Virtualization", RFC 7365, DOI 10.17487/RFC7365, October

2014, <http://www.rfc-editor.org/info/rfc7365>.

[RFC7637]  Garg, P., Ed. and Y. Wang, Ed., "NVGRE: Network

Virtualization Using Generic Routing Encapsulation",

RFC 7637, DOI 10.17487/RFC7637, September 2015,

<http://www.rfc-editor.org/info/rfc7637>.

[RFC7665]  Halpern, J., Ed. and C. Pignataro, Ed., "Service Function

Chaining (SFC) Architecture", RFC 7665,

DOI 10.17487/RFC7665, October 2015,

<http://www.rfc-editor.org/info/rfc7665>.

8.2.  Informative References

[I-D.ietf-bier-oam-requirements]

Mirsky, G., Nordmark, E., Pignataro, C., Kumar, N.,

Aldrin, S., Zheng, L., Chen, M., Akiya, N., and J.

Networks, "Operations, Administration and Maintenance

(OAM) Requirements for Bit Index Explicit Replication

(BIER) Layer", draft-ietf-bier-oam-requirements-00 (work

in progress), September 2015.

Authors' Addresses

Nagendra Kumar

Cisco Systems, Inc.

7200 Kit Creek Road

Research Triangle Park, NC  27709

US

Email: naikumar@cisco.com

Kumar, et al.          Expires September 13, 2016               [Page 8]

Internet-Draft          Overlay OAM Requirements              March 2016

Carlos Pignataro

Cisco Systems, Inc.

7200 Kit Creek Road

Research Triangle Park, NC  27709-4987

US

Email: cpignata@cisco.com

Deepak Kumar

Cisco Systems, Inc.

3700 Cisco Way

SJ

US

Email: dekumar@cisco.com

Greg Mirsky

Ericsson

Email: gregory.mirsky@ericsson.com

Mach Chen

Huawei Technologies

Email: mach.chen@huawei.com

Eric Nordmark

Arista Networks

Email: nordmark@acm.org

Santosh Pallagatti

Juniper Networks

Email: santosh.pallagatti@gmail.com

David Mozes

Mellanox Technologies Ltd

Email: davidm@mellanox.com

Kumar, et al.          Expires September 13, 2016               [Page 9]