Routing Area  Working Group                                    G. Mirsky

Internet-Draft                                                  Ericsson

Intended status: Informational                               E. Nordmark

Expires: December 14, 2016                               Arista Networks

C. Pignataro

N. Kumar

D. Kumar

Cisco Systems, Inc.

M. Chen

Y. Li

Huawei Technologies

D. Mozes

Mellanox Technologies Ltd.

S. Pallagatti

I. Bagdonas

June 12, 2016

Operations, Administration and Maintenance (OAM) for Overlay Networks:

Gap Analysis

draft-ooamdt-rtgwg-oam-gap-analysis-03

Abstract

This document provides an overview of the Operations, Administration,

and Maintenance (OAM) for overlay networks.  The OAM toolset includes

set of fault management and performance monitoring capabilities

(operating in the data plane) that comply with the Overlay OAM

Requirements.  Insufficient functional coverage of existing OAM

protocols also noted in this document.  The protocol definitions for

each of the Overlay OAM tools to be defined in separate documents.

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at http://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

Mirsky, et al.          Expires December 14, 2016               [Page 1]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

This Internet-Draft will expire on December 14, 2016.

Copyright Notice

Copyright (c) 2016 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents

(http://trustee.ietf.org/license-info) in effect on the date of

publication of this document.  Please review these documents

carefully, as they describe your rights and restrictions with respect

to this document.  Code Components extracted from this document must

include Simplified BSD License text as described in Section 4.e of

the Trust Legal Provisions and are provided without warranty as

described in the Simplified BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   3

1.1.  Conventions used in this document . . . . . . . . . . . .   4

1.1.1.  Terminology . . . . . . . . . . . . . . . . . . . . .   4

1.1.2.  Requirements Language . . . . . . . . . . . . . . . .   5

2.  Working Group Overview  . . . . . . . . . . . . . . . . . . .   5

2.1.  BIER  . . . . . . . . . . . . . . . . . . . . . . . . . .   5

2.2.  NVO3  . . . . . . . . . . . . . . . . . . . . . . . . . .   5

2.3.  SFC . . . . . . . . . . . . . . . . . . . . . . . . . . .   6

3.  Overlay OAM Toolset . . . . . . . . . . . . . . . . . . . . .   6

3.1.  Overlay OAM Fault Management  . . . . . . . . . . . . . .   6

3.1.1.  Proactive Continuity Check and Connectivity

Verification  . . . . . . . . . . . . . . . . . . . .   6

3.1.2.  On-demand Continuity Check and Connectivity

Verification  . . . . . . . . . . . . . . . . . . . .   8

3.1.3.  Alarm Indication Signal . . . . . . . . . . . . . . .   9

3.2.  Overlay OAM Performance Measurement . . . . . . . . . . .   9

3.2.1.  Overlay OAM PM Active . . . . . . . . . . . . . . . .   9

3.2.2.  Overlay OAM PM Passive  . . . . . . . . . . . . . . .  11

3.3.  Telemetry in Overlay OAM  . . . . . . . . . . . . . . . .  11

3.4.  Conclusions . . . . . . . . . . . . . . . . . . . . . . .  11

4.  IANA Considerations . . . . . . . . . . . . . . . . . . . . .  12

5.  Security Considerations . . . . . . . . . . . . . . . . . . .  12

6.  Acknowledgement . . . . . . . . . . . . . . . . . . . . . . .  12

7.  References  . . . . . . . . . . . . . . . . . . . . . . . . .  12

7.1.  Normative References  . . . . . . . . . . . . . . . . . .  12

7.2.  Informative References  . . . . . . . . . . . . . . . . .  12

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .  16

Mirsky, et al.          Expires December 14, 2016               [Page 2]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

1.  Introduction

Operations, Administration, and Maintenance (OAM) toolset provides

methods for fault management and performance monitoring in each layer

of the network, in order to improve their ability to support services

with guaranteed and strict Service Level Agreements (SLAs) while

reducing operational costs.

[RFC7276] provided detailed analysis of OAM protocols.  Since its

completion several new protocols that define data plane encapsulation

were introduced.  That presented both need to re-evaluate existing

set of OAM tools and opportunity to build it into set of tools that

can be used and re-used for different data plane protocols.

Overlay OAM Requirements define the set of requirements for OAM in

Overlay networks.  The OAM solution for Overlay networks, developed

by the design team, has two objectives:

o  The Overlay OAM toolset should be developed based on existing IP

and IP/MPLS architecture, technology, and toolsets.

o  The Overlay OAM operational experience should be similar to that

in other, e.g.  IP and IP/MPLS, networks.

The Overlay OAM toolset may use some or all of the following OAM

protocols designed at IETF:

o  proactive continuity check:

*  Bidirectional Forwarding Detection (BFD) for point-to-point as

defined in [RFC5880], [RFC5882], [RFC5883], [RFC5884],

[RFC5885], [RFC6428] and [RFC7726];

*  BFD for multipoint network as defined in

[I-D.ietf-bfd-multipoint] and

[I-D.ietf-bfd-multipoint-active-tail];

*  S-BFD as defined in [I-D.ietf-bfd-seamless-base] and

[I-D.ietf-bfd-seamless-ip];

o  on-demand continuity check and connectivity verification:

*  MPLS Echo Request/Reply, a.k.a.  LSP Ping, as defined in

[RFC4379] and its numerous extensions;

*  LSP Self-ping, as defined in [RFC7746];

Mirsky, et al.          Expires December 14, 2016               [Page 3]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

*  [I-D.kumarzheng-bier-ping] is a good example of generic

troubleshooting and defect localization tool that can be

extended and suited for more specific requirements of the

particular type of an overlay network;

o  performance measurement:

*  packet loss and delay measurement in MPLS networks, as defined

in [RFC6374] with ability to export measurement results for

post-processing [I-D.ietf-mpls-rfc6374-udp-return-path];

*  One-Way Active Measurement Protocol (OWAMP), as defined in

[RFC4656], and Two-Way Active Measurement Protocol (TWAMP), as

defined in [RFC5357], [RFC6038], and [RFC7750];

*  use of the Marking Method [I-D.tempia-ippm-p3m] that, if

accordingly supported by the overlay layer, can behave as close

as technically possible to a passive method to measure

performance, e.g.  [I-D.mirsky-bier-pmmm-oam].

1.1.  Conventions used in this document

1.1.1.  Terminology

Term "Overlay OAM" used in this document interchangeably with longer

version "set of OAM protocols, methods and tools for Overlay

networks".

AIS Alarm Indication Signal

BFD Bidirectional Forwarding Detection

BIER Bit-Indexed Explicit Replication

CC Continuity Check

CV Connectivity Verification

FM Fault Management

G-ACh Generic Associated Channel

Geneve Generic Network Virtualization Encapsulation

GUE Generic UDP Encapsulation

MPLS Multiprotocol Label Switching

Mirsky, et al.          Expires December 14, 2016               [Page 4]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

NTP Network Time Protocol

NVO3 Network Virtalization Overlays

OAM Operations, Administration, and Maintenance

OWAMP One-Way Active Measurement Protocol

PM Performance Measurement

PTP Precision Time Protocol

SFC Service Fundction Chaining

SFP Service Function Path

SLA Service Level Agreement

TWAMP Two-Way Active Measurement Protocol

VxLAN Virtual eXtensible Local Area Network

VxLAN-GPE Generic Protocol Extension for VxLAN

1.1.2.  Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and

"OPTIONAL" in this document are to be interpreted as described in

[RFC2119].

2.  Working Group Overview

2.1.  BIER

The BIER working group has some WG documents on OAM which are

discussed further in this document.

2.2.  NVO3

The NVO3 encapsulations (Geneve [I-D.ietf-nvo3-geneve], GUE

[I-D.ietf-nvo3-gue], and GPE [I-D.ietf-nvo3-vxlan-gpe]) all have some

notion of a OAM bit or flag.  In Geneve this is defined to not apply

to intermediate (underlay) routers and that the setting of the bit

doesn't affect the ECMP hash.  The other proposals do not have as

succinct constraints on their OAM bit/flag.

Mirsky, et al.          Expires December 14, 2016               [Page 5]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

There are currently no NVO3 working group OAM protocol

specifications.  The OAM documents that have been discussed are

individual drafts such as [I-D.ashwood-nvo3-oam-requirements],

[I-D.nordmark-nvo3-transcending-traceroute],

[I-D.pang-nvo3-vxlan-path-detection],

[I-D.saum-nvo3-pmtud-over-vxlan], and

[I-D.singh-nvo3-vxlan-router-alert].

2.3.  SFC

TBD

3.  Overlay OAM Toolset

It is expected that the encapsulation of an overlay network uses one

of methods discussed in [I-D.ietf-rtgwg-dt-encap] to distinctly

identify the payload as OAM, i.e. non-user, packet.  In its turn all

Overlay OAM protocols share the common Overlay OAM Header.  Format

and processing of the header are outside the scope of this document

and will be presented in the solution document.

3.1.  Overlay OAM Fault Management

Protocols that enable Fault Management functions of OAM toolset are

comprised of protocols that perform proactive and on-demand defect

detection and failure localization.

3.1.1.  Proactive Continuity Check and Connectivity Verification

Bidirectional Forwarding Detection (BFD) [RFC5880] is the protocol of

choice for proactive Continuity Check and Connectivity Verification

[RFC6428].

3.1.1.1.  Proactive CC/CV in BIER

.  Bit-Indexed Explicit Replication (BIER) provides the multicast

service.  For that BFD over multipoint network

[I-D.ietf-bfd-multipoint] and [I-D.ietf-bfd-multipoint-active-tail]

are the most suitable of BFD family Figure 1 presents IP/UDP format

of BFD over BIER in MPLS network.

Mirsky, et al.          Expires December 14, 2016               [Page 6]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              BIER-MPLS label          |     |1|               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 1 0 1|  Ver  |  Len  |              Entropy                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                BitString  (first 32 bits)                     ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                                                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                BitString  (last 32 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|OAM|     Reserved      | Proto |            BFIR-id            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                       IP Header                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        Source Port            |   Destination Port (3784)     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Length            |           Checksum            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                  BFD control packet                           ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1: BFD over BIER with IP/UDP format

Proto field MUST be set to IPv4 or IPv6 vlalue.  Note that IP

Destination address in Figure 1 must follow Section 7 [RFC5884], i.e.

?the destination IP address MUST be randomly chosen from the 127/8

range for IPv4 and from the 0:0:0:0:0:FFFF:7F00/104 range for IPv6.?

BFD packets in the reverse direction of the BFD session will be

transmitted on IP network to the IP address mapped to the BFIR-id and

the destination UDP port number set as source UDP port number of the

received BFD packet.

IP/UDP format presents overhead, particularly in case of IPv6 address

family.  Thus option to avoid use of extra headers for OAM seems

attractive.  Figure 2 presents G-ACh format of BFD over BIER in MPLS

network.  Proto field of the BIER header MUST be set to OAM value.

BFD control packet follows the BIER OAM header as defined in

[I-D.kumarzheng-bier-ping].  According to the Section 3.1 of

[I-D.kumarzheng-bier-ping], Ver is set to 1; BFD control packet over

multi-point without or with active tail accordingly identified in

Mirsky, et al.          Expires December 14, 2016               [Page 7]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

Message Type Field.  The Proto field ?is used to define if there is

any data packet immediately following the OAM payload?.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              BIER-MPLS label          |     |1|               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 1 0 1|  Ver  |  Len  |              Entropy                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                BitString  (first 32 bits)                     ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                                                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                BitString  (last 32 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|OAM|     Reserved    | Proto |             BFIR-id             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Ver | Message Type  | Proto |          Reserved               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                  BFD control packet                           ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2: BFD over BIER with G-ACh format

3.1.1.2.  Proactive CC/CV in NVO3

There is currently no WG document on proactive CC/CV.  The individual

requirements document [I-D.ashwood-nvo3-oam-requirements] covers this

and there is a related proposal for BFD over VXLAN in

[I-D.spallagatti-bfd-vxlan].

3.1.1.3.  Proactive CC/CV over SFP

3.1.2.  On-demand Continuity Check and Connectivity Verification

3.1.2.1.  On-demand CC/CV in BIER

[I-D.kumarzheng-bier-ping] defines format of Echo Request/Reply

control packet and set of TLVs that can be used to perform failure

detection and isolation in BIER domain over MPLS network.

Mirsky, et al.          Expires December 14, 2016               [Page 8]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

3.1.2.2.  On-demand CC/CV in NVO3

There is currently no WG document for on-demand CC/CV.

Individual documents exist for tracing such as

[I-D.pang-nvo3-vxlan-path-detection], and

[I-D.nordmark-nvo3-transcending-traceroute].

3.1.2.3.  On-demand CC/CV over SFP

3.1.3.  Alarm Indication Signal

3.1.3.1.  AIS in BIER

3.1.3.2.  AIS in NVO3

There is currently no WG document on Alarm Indication Signal.

The individual draft [I-D.nordmark-nvo3-transcending-traceroute]

suggests reusing ICMP errors for defect indications.

3.1.3.3.  AIS over SFP

3.2.  Overlay OAM Performance Measurement

3.2.1.  Overlay OAM PM Active

Requirements towards PM OAM for overlay networks are listed in the

Section 4.2 [I-D.ooamdt-rtgwg-ooam-requirement].  Two sets of

performance measurement protocols had been developed at IETF so far:

o  OWAMP [RFC4656] and TWAMP [RFC5357] each includes the control

protocol to negotiate required parameters and control a test

session as well as the test protocol itself that specify format

and processing of a test packet.  Historically, TWAMP, that

enables measurement of the latency, packet loss both as one-way

and round trip performance metric, has been implemented more often

and thus gained wider deployment than OWAMP.  There are several

properties of the test protocol that may not be suitable for its

use in overlay networks:

-  the test protocol is targetted to IP layer and carries some IP-

specific information;

-  the format of the sent test and the reflected packets differ

significantly and that complicates efficient HW-based

implementation;

Mirsky, et al.          Expires December 14, 2016               [Page 9]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

-  latency and packet loss measurement operations are bundled

together and that causes certain overhead when only one of

performance metrics is to be measured;

-  only Network Time Protocol (NTP) format of timestamp is

currently supported that requires additional processing to

convert from IEEE-1588 time format that broadly supported in

many current packet forwarding engines.

o  [RFC6374] defines the test protocol that enables measurement of

the latency and paket loss as one-way and round-trip perfomance

metrics.  Comparing to OWAMP/TWAMP RFC 6374 has certain

advantages:

-  the test protocol is flexible and these performance metrics can

be measured independently or in the single test session;

-  the protocol does not carry transport layer specific

information;

-  there's no difference between format of the packet transmitted

by the sender and reflected by the responder as the test

packets preallocates space for all necesary data it collects;

-  both NTP and PTP time formats allowed to be used to record time

for latency measurement.

[RFC6374] can be used as foundation of active PM OAM in overlay

networks.  The YANG data model [RFC6020] of the packet loss and delay

measurement based on [RFC6374] can improve control and increase

operational value of active performance measurement in overlay

networks.

3.2.1.1.  Active PM in BIER

Currently there is no draft related to active PM OAM in the WG.

3.2.1.2.  Active PM in NVO3

Performance management has been discussed in NVO3 but there is

currently no draft in the WG.

3.2.1.3.  Active PM over SFP

Mirsky, et al.          Expires December 14, 2016              [Page 10]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

3.2.2.  Overlay OAM PM Passive

3.2.2.1.  Passive PM in BIER

[I-D.mirsky-bier-pmmm-oam] describes how the Marking Method can be

used in BIER domain over MPLS networks.

3.2.2.2.  Passive PM in NVO3

Marking has been discussed in NVO3 sessions, but there is no draft in

the working group.

3.2.2.3.  Passive PM over SFP

3.3.  Telemetry in Overlay OAM

Excessive use of the in-band OAM channel may affect user flow and

thus change network behavior.  For example, if operator uses passive

measurement exporting massive amount of data over the OAM channel may

affect network.  I think that a management channel should be used in

such case.  Obviously it may traverse the same nodes and links but

may not require the same QoS.  We can refer to LMAP Reference Model

[RFC7594] with Controller, Measurement Agent and Data Collector.

[I-D.lapukhov-dataplane-probe] proposes transport independent generic

telemetry probe structure.

3.4.  Conclusions

o  A common Overlay OAM header should be defined to support

demultiplexing of OAM protocols.

o  Existing modes of BFD protocol, primarily its Async mode, can be

used either in IP/UDP or ACH format, as proactive continuity check

mechanism in overlay networks.

o  A new control packet to be used for on-demand CC/CV in overlay

networks should be defined.  Set of common TLVs may be defined

while more specific TLVs to be defined by respective groups of

experts.

o  [RFC6374] can be used as the foundation of active performance

measurement OAM in overlay networks.

o  YANG data model of the active PM OAM in overlay networks would

improve control and increase operational value of the test

methods.

Mirsky, et al.          Expires December 14, 2016              [Page 11]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

4.  IANA Considerations

This document does not propose any IANA consideration.  This section

may be removed.

5.  Security Considerations

6.  Acknowledgement

TBD

7.  References

7.1.  Normative References

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119,

DOI 10.17487/RFC2119, March 1997,

<http://www.rfc-editor.org/info/rfc2119>.

7.2.  Informative References

[I-D.ashwood-nvo3-oam-requirements]

Chen, H., Ashwood-Smith, P., Xia, L., Iyengar, R., Tsou,

T., Sajassi, A., Boucadair, M., Jacquenet, C., Daikoku,

M., Ghanwani, A., and R. Krishnan, "NVO3 Operations,

Administration, and Maintenance Requirements", draft-

ashwood-nvo3-oam-requirements-04 (work in progress),

October 2015.

[I-D.ietf-bfd-multipoint]

Katz, D., Ward, D., and J. Networks, "BFD for Multipoint

Networks", draft-ietf-bfd-multipoint-07 (work in

progress), August 2015.

[I-D.ietf-bfd-multipoint-active-tail]

Katz, D., Ward, D., and J. Networks, "BFD Multipoint

Active Tails.", draft-ietf-bfd-multipoint-active-tail-01

(work in progress), November 2015.

[I-D.ietf-bfd-seamless-base]

Akiya, N., Pignataro, C., Ward, D., Bhatia, M., and J.

Networks, "Seamless Bidirectional Forwarding Detection

(S-BFD)", draft-ietf-bfd-seamless-base-08 (work in

progress), February 2016.

Mirsky, et al.          Expires December 14, 2016              [Page 12]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

[I-D.ietf-bfd-seamless-ip]

Akiya, N., Pignataro, C., and D. Ward, "Seamless

Bidirectional Forwarding Detection (S-BFD) for IPv4, IPv6

and MPLS", draft-ietf-bfd-seamless-ip-03 (work in

progress), February 2016.

[I-D.ietf-mpls-rfc6374-udp-return-path]

Bryant, S., Sivabalan, S., and S. Soni, "RFC6374 UDP

Return Path", draft-ietf-mpls-rfc6374-udp-return-path-04

(work in progress), August 2015.

[I-D.ietf-nvo3-geneve]

Gross, J. and I. Ganga, "Geneve: Generic Network

Virtualization Encapsulation", draft-ietf-nvo3-geneve-01

(work in progress), January 2016.

[I-D.ietf-nvo3-gue]

Herbert, T., Yong, L., and O. Zia, "Generic UDP

Encapsulation", draft-ietf-nvo3-gue-02 (work in progress),

December 2015.

[I-D.ietf-nvo3-vxlan-gpe]

Quinn, P., Manur, R., Kreeger, L., Lewis, D., Maino, F.,

Smith, M., Agarwal, P., Yong, L., Xu, X., Elzur, U., Garg,

P., and D. Melman, "Generic Protocol Extension for VXLAN",

draft-ietf-nvo3-vxlan-gpe-01 (work in progress), November

2015.

[I-D.ietf-rtgwg-dt-encap]

Nordmark, E., Tian, A., Gross, J., Hudson, J., Kreeger,

L., Garg, P., Thaler, P., and T. Herbert, "Encapsulation

Considerations", draft-ietf-rtgwg-dt-encap-01 (work in

progress), March 2016.

[I-D.kumarzheng-bier-ping]

Kumar, N., Pignataro, C., Akiya, N., Zheng, L., Chen, M.,

and G. Mirsky, "BIER Ping and Trace", draft-kumarzheng-

bier-ping-02 (work in progress), December 2015.

[I-D.lapukhov-dataplane-probe]

Lapukhov, P., "Data-plane probe for in-band telemetry

collection", draft-lapukhov-dataplane-probe-00 (work in

progress), March 2016.

Mirsky, et al.          Expires December 14, 2016              [Page 13]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

[I-D.mirsky-bier-pmmm-oam]

Mirsky, G., Zheng, L., Chen, M., and G. Fioccola,

"Performance Measurement (PM) with Marking Method in Bit

Index Explicit Replication (BIER) Layer", draft-mirsky-

bier-pmmm-oam-01 (work in progress), March 2016.

[I-D.nordmark-nvo3-transcending-traceroute]

Nordmark, E., Appanna, C., and A. Lo, "Layer-Transcending

Traceroute for Overlay Networks like VXLAN", draft-

nordmark-nvo3-transcending-traceroute-02 (work in

progress), March 2016.

[I-D.ooamdt-rtgwg-ooam-requirement]

Kumar, N., Pignataro, C., Kumar, D., Mirsky, G., Chen, M.,

Nordmark, E., Networks, J., and D. Mozes, "Overlay OAM

Requirements", draft-ooamdt-rtgwg-ooam-requirement-00

(work in progress), March 2016.

[I-D.pang-nvo3-vxlan-path-detection]

<>, J., Liu, D., Liu, D., Zhang, D., Yizhou, L., Chen, H.,

<>, D., <>, B., Kumar, D., Gao, R., and Y. Qiao, "Path

Detection in VXLAN Overlay Network", draft-pang-nvo3-

vxlan-path-detection-02 (work in progress), March 2016.

[I-D.saum-nvo3-pmtud-over-vxlan]

Dikshit, S. and N. A, "PMTUD Over Vxlan", draft-saum-nvo3-

pmtud-over-vxlan-02 (work in progress), December 2015.

[I-D.singh-nvo3-vxlan-router-alert]

Singh, K., Jain, P., Rio, D., Henderickx, W., Shekhar, R.,

and R. Rahman, "VxLAN Router Alert Option", draft-singh-

nvo3-vxlan-router-alert-02 (work in progress), September

2015.

[I-D.spallagatti-bfd-vxlan]

Networks, J., Basil, B., Paragiri, S., Govindan, V.,

Mudigonda, M., and G. Mirsky, "BFD for VXLAN", draft-

spallagatti-bfd-vxlan-02 (work in progress), October 2015.

[I-D.tempia-ippm-p3m]

Capello, A., Cociglio, M., Fioccola, G., Castaldelli, L.,

and A. Bonda, "A packet based method for passive

performance monitoring", draft-tempia-ippm-p3m-03 (work in

progress), March 2016.

Mirsky, et al.          Expires December 14, 2016              [Page 14]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

[RFC4379]  Kompella, K. and G. Swallow, "Detecting Multi-Protocol

Label Switched (MPLS) Data Plane Failures", RFC 4379,

DOI 10.17487/RFC4379, February 2006,

<http://www.rfc-editor.org/info/rfc4379>.

[RFC4656]  Shalunov, S., Teitelbaum, B., Karp, A., Boote, J., and M.

Zekauskas, "A One-way Active Measurement Protocol

(OWAMP)", RFC 4656, DOI 10.17487/RFC4656, September 2006,

<http://www.rfc-editor.org/info/rfc4656>.

[RFC5357]  Hedayat, K., Krzanowski, R., Morton, A., Yum, K., and J.

Babiarz, "A Two-Way Active Measurement Protocol (TWAMP)",

RFC 5357, DOI 10.17487/RFC5357, October 2008,

<http://www.rfc-editor.org/info/rfc5357>.

[RFC5880]  Katz, D. and D. Ward, "Bidirectional Forwarding Detection

(BFD)", RFC 5880, DOI 10.17487/RFC5880, June 2010,

<http://www.rfc-editor.org/info/rfc5880>.

[RFC5882]  Katz, D. and D. Ward, "Generic Application of

Bidirectional Forwarding Detection (BFD)", RFC 5882,

DOI 10.17487/RFC5882, June 2010,

<http://www.rfc-editor.org/info/rfc5882>.

[RFC5883]  Katz, D. and D. Ward, "Bidirectional Forwarding Detection

(BFD) for Multihop Paths", RFC 5883, DOI 10.17487/RFC5883,

June 2010, <http://www.rfc-editor.org/info/rfc5883>.

[RFC5884]  Aggarwal, R., Kompella, K., Nadeau, T., and G. Swallow,

"Bidirectional Forwarding Detection (BFD) for MPLS Label

Switched Paths (LSPs)", RFC 5884, DOI 10.17487/RFC5884,

June 2010, <http://www.rfc-editor.org/info/rfc5884>.

[RFC5885]  Nadeau, T., Ed. and C. Pignataro, Ed., "Bidirectional

Forwarding Detection (BFD) for the Pseudowire Virtual

Circuit Connectivity Verification (VCCV)", RFC 5885,

DOI 10.17487/RFC5885, June 2010,

<http://www.rfc-editor.org/info/rfc5885>.

[RFC6020]  Bjorklund, M., Ed., "YANG - A Data Modeling Language for

the Network Configuration Protocol (NETCONF)", RFC 6020,

DOI 10.17487/RFC6020, October 2010,

<http://www.rfc-editor.org/info/rfc6020>.

[RFC6038]  Morton, A. and L. Ciavattone, "Two-Way Active Measurement

Protocol (TWAMP) Reflect Octets and Symmetrical Size

Features", RFC 6038, DOI 10.17487/RFC6038, October 2010,

<http://www.rfc-editor.org/info/rfc6038>.

Mirsky, et al.          Expires December 14, 2016              [Page 15]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

[RFC6374]  Frost, D. and S. Bryant, "Packet Loss and Delay

Measurement for MPLS Networks", RFC 6374,

DOI 10.17487/RFC6374, September 2011,

<http://www.rfc-editor.org/info/rfc6374>.

[RFC6428]  Allan, D., Ed., Swallow, G., Ed., and J. Drake, Ed.,

"Proactive Connectivity Verification, Continuity Check,

and Remote Defect Indication for the MPLS Transport

Profile", RFC 6428, DOI 10.17487/RFC6428, November 2011,

<http://www.rfc-editor.org/info/rfc6428>.

[RFC7276]  Mizrahi, T., Sprecher, N., Bellagamba, E., and Y.

Weingarten, "An Overview of Operations, Administration,

and Maintenance (OAM) Tools", RFC 7276,

DOI 10.17487/RFC7276, June 2014,

<http://www.rfc-editor.org/info/rfc7276>.

[RFC7594]  Eardley, P., Morton, A., Bagnulo, M., Burbridge, T.,

Aitken, P., and A. Akhter, "A Framework for Large-Scale

Measurement of Broadband Performance (LMAP)", RFC 7594,

DOI 10.17487/RFC7594, September 2015,

<http://www.rfc-editor.org/info/rfc7594>.

[RFC7726]  Govindan, V., Rajaraman, K., Mirsky, G., Akiya, N., and S.

Aldrin, "Clarifying Procedures for Establishing BFD

Sessions for MPLS Label Switched Paths (LSPs)", RFC 7726,

DOI 10.17487/RFC7726, January 2016,

<http://www.rfc-editor.org/info/rfc7726>.

[RFC7746]  Bonica, R., Minei, I., Conn, M., Pacella, D., and L.

Tomotaki, "Label Switched Path (LSP) Self-Ping", RFC 7746,

DOI 10.17487/RFC7746, January 2016,

<http://www.rfc-editor.org/info/rfc7746>.

[RFC7750]  Hedin, J., Mirsky, G., and S. Baillargeon, "Differentiated

Service Code Point and Explicit Congestion Notification

Monitoring in the Two-Way Active Measurement Protocol

(TWAMP)", RFC 7750, DOI 10.17487/RFC7750, February 2016,

<http://www.rfc-editor.org/info/rfc7750>.

Authors' Addresses

Greg Mirsky

Ericsson

Email: gregory.mirsky@ericsson.com

Mirsky, et al.          Expires December 14, 2016              [Page 16]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

Erik Nordmark

Arista Networks

Email: nordmark@acm.org

Carlos Pignataro

Cisco Systems, Inc.

Email: cpignata@cisco.com

Nagendra Kumar

Cisco Systems, Inc.

Email: naikumar@cisco.com

Deepak Kumar

Cisco Systems, Inc.

Email: dekumar@cisco.com

Mach Chen

Huawei Technologies

Email: mach.chen@huawei.com

Yizhou Li

Huawei Technologies

Email: liyizhou@huawei.com

David Mozes

Mellanox Technologies Ltd.

Email: davidm@mellanox.com

Santosh Pallagatti

Email: santosh.pallagatti@gmail.com

Mirsky, et al.          Expires December 14, 2016              [Page 17]

Internet-Draft       OAM for Overlays: Gap Analysis            June 2016

Ignas Bagdonas

Email: ibagdona@gmail.com

Mirsky, et al.          Expires December 14, 2016              [Page 18]