Routing Area  Working Group                                    G. Mirsky

Internet-Draft                                                  Ericsson

Intended status: Informational                               E. Nordmark

Expires: September 21, 2016                              Arista Networks

C. Pignataro

N. Kumar

D. Kumar

Cisco Systems, Inc.

M. Chen

Huawei Technologies

D. Mozes

Mellanox Technologies Ltd.

S. Pallagatti

March 20, 2016

Operations, Administration and Maintenance (OAM) for Overlay Networks:

Gap Analysis

draft-ooamdt-rtgwg-oam-gap-analysis-00

Abstract

This document provides an overview of the Operations, Administration,

and Maintenance (OAM) for overlay networks.  The OAM toolset includes

set of fault management and performance monitoring capabilities

(operating in the data plane) that comply with the Overlay OAM

Requirements.  Insufficient functional coverage of existing OAM

protocols also noted in this document.  The protocol definitions for

each of the Overlay OAM tools to be defined in separate documents.

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at http://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on September 21, 2016.

Mirsky, et al.         Expires September 21, 2016               [Page 1]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

Copyright Notice

Copyright (c) 2016 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents

(http://trustee.ietf.org/license-info) in effect on the date of

publication of this document.  Please review these documents

carefully, as they describe your rights and restrictions with respect

to this document.  Code Components extracted from this document must

include Simplified BSD License text as described in Section 4.e of

the Trust Legal Provisions and are provided without warranty as

described in the Simplified BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   2

1.1.  Conventions used in this document . . . . . . . . . . . .   4

1.1.1.  Terminology . . . . . . . . . . . . . . . . . . . . .   4

1.1.2.  Requirements Language . . . . . . . . . . . . . . . .   4

2.  Overlay OAM Toolset . . . . . . . . . . . . . . . . . . . . .   4

2.1.  Overlay OAM Fault Management  . . . . . . . . . . . . . .   4

2.1.1.  Proactive Continuity Check and Connectivity

Verification  . . . . . . . . . . . . . . . . . . . .   4

2.1.2.  On-demand Continuity Check and Connectivity

Verification  . . . . . . . . . . . . . . . . . . . .   6

2.1.3.  Alarm Indication Signal . . . . . . . . . . . . . . .   7

2.2.  Overlay OAM Performance Measurement . . . . . . . . . . .   7

2.2.1.  Overlay OAM PM Active . . . . . . . . . . . . . . . .   7

2.2.2.  Overlay OAM PM Passive  . . . . . . . . . . . . . . .   7

2.3.  Telemetry in Overlay OAM  . . . . . . . . . . . . . . . .   7

2.4.  Conclusions . . . . . . . . . . . . . . . . . . . . . . .   7

3.  IANA Considerations . . . . . . . . . . . . . . . . . . . . .   7

4.  Security Considerations . . . . . . . . . . . . . . . . . . .   7

5.  Acknowledgement . . . . . . . . . . . . . . . . . . . . . . .   8

6.  References  . . . . . . . . . . . . . . . . . . . . . . . . .   8

6.1.  Normative References  . . . . . . . . . . . . . . . . . .   8

6.2.  Informative References  . . . . . . . . . . . . . . . . .   8

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .  10

1.  Introduction

Operations, Administration, and Maintenance (OAM) toolset provides

methods for fault management and performance monitoring in each layer

of the network, in order to improve their ability to support services

with guaranteed and strict Service Level Agreements (SLAs) while

reducing operational costs.

Mirsky, et al.         Expires September 21, 2016               [Page 2]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

[RFC7276] provided detailed analysis of OAM protocols.  Since its

completion several new protocols that define data plane encapsulation

were introduced.  That presented both need to re-evaluate existing

set of OAM tools and opportunity to build it into set of tools that

can be used and re-used for different data plane protocols.

Overlay OAM Requirements define the set of requirements for OAM in

Overlay networks.  The OAM solution for Overlay networks, developed

by the design team, has two objectives:

o  The Overlay OAM toolset should be developed based on existing IP

and IP/MPLS architecture, technology, and toolsets.

o  The Overlay OAM operational experience should be similar to that

in other, e.g.  IP and IP/MPLS, networks.

The Overlay OAM toolset may use some or all of the following OAM

protocols designed at IETF:

o  proactive continuity check:

*  Bidirectional Forwarding Detection (BFD) for point-to-point as

defined in [RFC5880], [RFC5882], [RFC5883], [RFC5884],

[RFC5885], [RFC6428] and [RFC7726];

*  BFD for multipoint network as defined in

[I-D.ietf-bfd-multipoint] and

[I-D.ietf-bfd-multipoint-active-tail];

*  S-BFD as defined in [I-D.ietf-bfd-seamless-base] and

[I-D.ietf-bfd-seamless-ip];

o  on-demand continuity check and connectivity verification:

*  MPLS Echo Request/Reply, a.k.a.  LSP Ping, as defined in

[RFC4379] and its numerous extensions;

*  LSP Self-ping, as defined in [RFC7746];

*  [I-D.kumarzheng-bier-ping] is a good example of generic

troubleshooting and defect localization tool that can be

extended and suited for more specific requirements of the

particular type of an overlay network;

o  performance measurement:

*  packet loss and delay measurement in MPLS networks, as defined

in [RFC6374];

Mirsky, et al.         Expires September 21, 2016               [Page 3]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

*  Two-Way Active Measurement Protocol (TWAMP), as defined in

[RFC5357], [RFC6038], and [RFC7750];

1.1.  Conventions used in this document

1.1.1.  Terminology

Term "Overlay OAM" used in this document interchangeably with longer

version "set of OAM protocols, methods and tools for Overlay

networks".

BFD Bidirectional Forwarding Detection

FM Fault Management

OAM Operations, Administration, and Maintenance

PM Performance Measurement

SLA Service Level Agreement

TWAMP Two-Way Active Measurement Protocol

1.1.2.  Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and

"OPTIONAL" in this document are to be interpreted as described in

[RFC2119].

2.  Overlay OAM Toolset

2.1.  Overlay OAM Fault Management

Protocols that enable Fault Management functions of OAM toolset are

comprised of protocols that perform proactive and on-demand defect

detection and failure localization.

2.1.1.  Proactive Continuity Check and Connectivity Verification

Bidirectional Forwarding Detection (BFD) [RFC5880] is the protocol of

choice for proactive Continuity Check and Connectivity Verification

[RFC6428].

Mirsky, et al.         Expires September 21, 2016               [Page 4]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

2.1.1.1.  Proactive CC/CV in BIER

.  Bit-Indexed Explicit Replication (BIER) provides the multicast

service.  Figure 1 presents IP/UDP format of BFD over BIER in MPLS

network.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              BIER-MPLS label          |     |1|               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 1 0 1|  Ver  |  Len  |              Entropy                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                BitString  (first 32 bits)                     ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                                                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                BitString  (last 32 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|OAM|     Reserved      | Proto |            BFIR-id            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                       IP Header                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        Source Port            |   Destination Port (3784)     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             Length            |           Checksum            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                  BFD control packet                           ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1: BFD over BIER with IP/UDP format

Proto field MUST be set to IPv4 or IPv6 vlalue.  Note that IP

Destination address in Figure 1 must follow Section 7 [RFC5884], i.e.

?the destination IP address MUST be randomly chosen from the 127/8

range for IPv4 and from the 0:0:0:0:0:FFFF:7F00/104 range for IPv6.?

BFD packets in the reverse direction of the BFD session will be

transmitted on IP network to the IP address mapped to the BFIR-id and

the destination UDP port number set as source UDP port number of the

recieved BFD packet.

IP/UDP format presents overhead, particularly in case of IPv6 address

family.  Thus option to avoid use of extra headers for OAM seems

attractive.  Figure 2 presents G-ACh format of BFD over BIER in MPLS

Mirsky, et al.         Expires September 21, 2016               [Page 5]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

network.  Proto field of the BIER header MUST be set to OAM value.

BFD control packet follows the BIER OAM header as defined in

[I-D.kumarzheng-bier-ping].  According to the Section 3.1 of

[I-D.kumarzheng-bier-ping], Ver is set to 1; BFD control packet

identified in Message Type Field.  The Proto field ?is used to define

if there is any data packet immediately following the OAM payload?.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Label Stack Element                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              BIER-MPLS label          |     |1|               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0 1 0 1|  Ver  |  Len  |              Entropy                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                BitString  (first 32 bits)                     ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                                                               ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                BitString  (last 32 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|OAM|     Reserved    | Proto |             BFIR-id             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Ver | Message Type  | Proto |          Reserved               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

~                  BFD control packet                           ~

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2: BFD over BIER with G-ACh format

In the case when G-ACh format of BFD used the reverse direction per

[RFC5884] will go to the IP address mapped to BFIR-id but identifying

Destination Port number is problematic.  This can be addressed by

bootstraping BFD session using Echo Request similarly to [RFC5884].

2.1.1.2.  Proactive CC/CV in NVO3

2.1.1.3.  Proactive CC/CV over SFP

2.1.2.  On-demand Continuity Check and Connectivity Verification

Mirsky, et al.         Expires September 21, 2016               [Page 6]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

2.1.2.1.  On-demand CC/CV in BIER

2.1.2.2.  On-demand CC/CV in NVO3

2.1.2.3.  On-demand CC/CV over SFP

2.1.3.  Alarm Indication Signal

2.1.3.1.  AIS in BIER

2.1.3.2.  AIS in NVO3

2.1.3.3.  AIS over SFP

2.2.  Overlay OAM Performance Measurement

2.2.1.  Overlay OAM PM Active

2.2.1.1.  Active PM in BIER

2.2.1.2.  Active PM in NVO3

2.2.1.3.  Active PM over SFP

2.2.2.  Overlay OAM PM Passive

2.2.2.1.  Passive PM in BIER

2.2.2.2.  Passive PM in NVO3

2.2.2.3.  Passive PM over SFP

2.3.  Telemetry in Overlay OAM

2.4.  Conclusions

3.  IANA Considerations

This document does not propose any IANA consideration.  This section

may be removed.

4.  Security Considerations

This document list the OAM requirement for BIER-enabled domain and

does not raise any security concerns or issues in addition to ones

common to networking.

Mirsky, et al.         Expires September 21, 2016               [Page 7]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

5.  Acknowledgement

TBD

6.  References

6.1.  Normative References

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119,

DOI 10.17487/RFC2119, March 1997,

<http://www.rfc-editor.org/info/rfc2119>.

6.2.  Informative References

[I-D.ietf-bfd-multipoint]

Katz, D., Ward, D., and J. Networks, "BFD for Multipoint

Networks", draft-ietf-bfd-multipoint-07 (work in

progress), August 2015.

[I-D.ietf-bfd-multipoint-active-tail]

Katz, D., Ward, D., and J. Networks, "BFD Multipoint

Active Tails.", draft-ietf-bfd-multipoint-active-tail-01

(work in progress), November 2015.

[I-D.ietf-bfd-seamless-base]

Akiya, N., Pignataro, C., Ward, D., Bhatia, M., and J.

Networks, "Seamless Bidirectional Forwarding Detection

(S-BFD)", draft-ietf-bfd-seamless-base-08 (work in

progress), February 2016.

[I-D.ietf-bfd-seamless-ip]

Akiya, N., Pignataro, C., and D. Ward, "Seamless

Bidirectional Forwarding Detection (S-BFD) for IPv4, IPv6

and MPLS", draft-ietf-bfd-seamless-ip-03 (work in

progress), February 2016.

[I-D.kumarzheng-bier-ping]

Kumar, N., Pignataro, C., Akiya, N., Zheng, L., Chen, M.,

and G. Mirsky, "BIER Ping and Trace", draft-kumarzheng-

bier-ping-02 (work in progress), December 2015.

[RFC4379]  Kompella, K. and G. Swallow, "Detecting Multi-Protocol

Label Switched (MPLS) Data Plane Failures", RFC 4379,

DOI 10.17487/RFC4379, February 2006,

<http://www.rfc-editor.org/info/rfc4379>.

Mirsky, et al.         Expires September 21, 2016               [Page 8]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

[RFC5357]  Hedayat, K., Krzanowski, R., Morton, A., Yum, K., and J.

Babiarz, "A Two-Way Active Measurement Protocol (TWAMP)",

RFC 5357, DOI 10.17487/RFC5357, October 2008,

<http://www.rfc-editor.org/info/rfc5357>.

[RFC5880]  Katz, D. and D. Ward, "Bidirectional Forwarding Detection

(BFD)", RFC 5880, DOI 10.17487/RFC5880, June 2010,

<http://www.rfc-editor.org/info/rfc5880>.

[RFC5882]  Katz, D. and D. Ward, "Generic Application of

Bidirectional Forwarding Detection (BFD)", RFC 5882,

DOI 10.17487/RFC5882, June 2010,

<http://www.rfc-editor.org/info/rfc5882>.

[RFC5883]  Katz, D. and D. Ward, "Bidirectional Forwarding Detection

(BFD) for Multihop Paths", RFC 5883, DOI 10.17487/RFC5883,

June 2010, <http://www.rfc-editor.org/info/rfc5883>.

[RFC5884]  Aggarwal, R., Kompella, K., Nadeau, T., and G. Swallow,

"Bidirectional Forwarding Detection (BFD) for MPLS Label

Switched Paths (LSPs)", RFC 5884, DOI 10.17487/RFC5884,

June 2010, <http://www.rfc-editor.org/info/rfc5884>.

[RFC5885]  Nadeau, T., Ed. and C. Pignataro, Ed., "Bidirectional

Forwarding Detection (BFD) for the Pseudowire Virtual

Circuit Connectivity Verification (VCCV)", RFC 5885,

DOI 10.17487/RFC5885, June 2010,

<http://www.rfc-editor.org/info/rfc5885>.

[RFC6038]  Morton, A. and L. Ciavattone, "Two-Way Active Measurement

Protocol (TWAMP) Reflect Octets and Symmetrical Size

Features", RFC 6038, DOI 10.17487/RFC6038, October 2010,

<http://www.rfc-editor.org/info/rfc6038>.

[RFC6374]  Frost, D. and S. Bryant, "Packet Loss and Delay

Measurement for MPLS Networks", RFC 6374,

DOI 10.17487/RFC6374, September 2011,

<http://www.rfc-editor.org/info/rfc6374>.

[RFC6428]  Allan, D., Ed., Swallow, G., Ed., and J. Drake, Ed.,

"Proactive Connectivity Verification, Continuity Check,

and Remote Defect Indication for the MPLS Transport

Profile", RFC 6428, DOI 10.17487/RFC6428, November 2011,

<http://www.rfc-editor.org/info/rfc6428>.

Mirsky, et al.         Expires September 21, 2016               [Page 9]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

[RFC7276]  Mizrahi, T., Sprecher, N., Bellagamba, E., and Y.

Weingarten, "An Overview of Operations, Administration,

and Maintenance (OAM) Tools", RFC 7276,

DOI 10.17487/RFC7276, June 2014,

<http://www.rfc-editor.org/info/rfc7276>.

[RFC7726]  Govindan, V., Rajaraman, K., Mirsky, G., Akiya, N., and S.

Aldrin, "Clarifying Procedures for Establishing BFD

Sessions for MPLS Label Switched Paths (LSPs)", RFC 7726,

DOI 10.17487/RFC7726, January 2016,

<http://www.rfc-editor.org/info/rfc7726>.

[RFC7746]  Bonica, R., Minei, I., Conn, M., Pacella, D., and L.

Tomotaki, "Label Switched Path (LSP) Self-Ping", RFC 7746,

DOI 10.17487/RFC7746, January 2016,

<http://www.rfc-editor.org/info/rfc7746>.

[RFC7750]  Hedin, J., Mirsky, G., and S. Baillargeon, "Differentiated

Service Code Point and Explicit Congestion Notification

Monitoring in the Two-Way Active Measurement Protocol

(TWAMP)", RFC 7750, DOI 10.17487/RFC7750, February 2016,

<http://www.rfc-editor.org/info/rfc7750>.

Authors' Addresses

Greg Mirsky

Ericsson

Email: gregory.mirsky@ericsson.com

Erik Nordmark

Arista Networks

Email: nordmark@acm.org

Carlos Pignataro

Cisco Systems, Inc.

Email: cpignata@cisco.com

Nagendra Kumar

Cisco Systems, Inc.

Email: naikumar@cisco.com

Mirsky, et al.         Expires September 21, 2016              [Page 10]

Internet-Draft       OAM for Overlays: Gap Analysis           March 2016

Deepak Kumar

Cisco Systems, Inc.

Email: dekumar@cisco.com

Mach Chen

Huawei Technologies

Email: mach.chen@huawei.com

David Mozes

Mellanox Technologies Ltd.

Email: davidm@mellanox.com

Santosh Pallagatti

Email: santosh.pallagatti@gmail.com

Mirsky, et al.         Expires September 21, 2016              [Page 11]